Article

# Reliability Assessment of TEC Maps over Brazil: A Multi-Source Comparative Study with Ground Validation Reliability Assessment of Multi-Source TEC Maps over Brazil Using Ground Truth Validation

Marco A. de U. Cintra 1,*, Stephan Stephany 1, Lamartine N. F. Guimarães ², Eurico R. de Paula ³, 
André R. F. Martinon ³, Patrícia M. de S. Negreti ³, Alison de O. Moraes 4,5 and Jonas R. de Souza ³

Academic Editor: Firstname Lastname Received: date Revised: date Accepted: date Published: date Citation: To be added by editorial staff during production. Copyright: © 2025 by the authors. Submitted for possible open access publication under the terms and conditions of the Creative Commons Attribution (CC BY) license (https://creativecommons.org/licenses/by/4.0/).

1	Instituto Nacional de Pesquisas Espaciais - INPE / Coordenação-Geral de Infraestrutura e Pesquisas Aplicadas - CGIP / Coordenação de Pesquisa Aplicada e Desenvolvimento Tecnológico - COPDT / Programa de Pós-Graduacão em Computação Aplicada (PG-CAP) - São José dos Campos/SP - Brazil; stephan.stephany@inpe.br

2	Instituto Tecnológico de Aeronáutica - ITA / Programa de Pós-Graduação em Ciências e Tecnologias Espaciais (PG-CTE) - São José dos Campos/SP - Brazil; lamartine.guimaraes@gp.ita.br

3   Instituto Nacional de Pesquisas Espaciais - INPE / Coordenação-Geral de Engenharia, Tecnologia e Ciência Espaciais - CGCE / Divisão de Heliofísica, Ciências Planetárias e Aeronomia - DIHPA - São José dos Campos/SP - Brazil; eurico.paula@inpe.br (E.R.P.); andre.martinon@inpe.br (A.R.F.M.); paty.mara@gmail.com (P.M.S.N.); jonas.souza@inpe.br (J.R.S.)

4   Instituto Tecnológico de Aeronáutica - ITA - São José dos Campos/SP - Brazil; aom@ita.br

5   Universidade de Taubaté – Unitau – Taubaté/SP – Brazil

*	Correspondence: marco.cintra@inpe.br

Abstract: Total Electron Content (TEC) maps allow us to evaluate the state of the ionosphere. There are many providers/sources of worldwide or regional TEC maps for the continuous monitoring of the ionosphere, but employing different GNSS monitoring networks for data acquisition, TEC calculation or interpolation methods for generating the maps, or different spatial and temporal resolutions and coverage. How reliable are TEC maps over Brazil? We employed TEC maps of four providers for 2022-2024, in the growing phase of the current solar cycle 25. Seasonality is also taken into account. A systematic comparison of TEC maps over Brazil was done using correlation and similarity analysis between maps of different sources. Significant differences were found. Even for the same source there are differences in the density of monitoring stations according to the region. An example of bubble signature in TEC maps is also analyzed. Ground truth validation of TEC is performed by comparing TEC point values extracted from the maps with values derived from a set of GNSS stations over Brazil. As a result, no TEC maps of these sources were deemed reliable, due to poor low spatial and/or temporal resolution, low monitoring station density or inadequate interpolation scheme.

Keywords: total electron content; TEC maps; TEC measurements; ionospheric irregularities.

# 1. Introduction

The Total Electron Content (TEC) field allows to evaluate and model the state of the ionosphere, which is affected by the space weather. Two-dimensional TEC maps are generated using vertical TEC values derived from sets of GNSS stations. Time series of TEC maps are intended to identify the formation and the evolution of ionospheric bubbles over time. There are many methodologies [1-10] being employed by scientific institutes or governmental organizations for the continuous monitoring of the ionosphere by means of TEC maps, here denoted as TEC map sources or providers. Differences between maps are expected since sources may employ different GNSS monitoring networks for data acquisition, TEC calculation methods or  interpolation methods for generating the maps. In addition, TEC maps sources provide maps with different spatial and temporal resolution. There is an abundance of scientific works using TEC maps, but usually employing a single source, which is assumed as the most convenient. Because of the variety of TEC map sources, understanding how consistent and accurate such maps are, especially over low-latitude regions like Brazil, seems to be important.

Considering TEC maps over Brazil, how reliable are the TEC maps of different sources/providers? This work aims to provide a systematic comparison of TEC maps from four major sources by statistical correlation and image similarity analysis, and in addition, performing a validation with ground truth using a set of GNSS stations. It tries to suggest which one, if any,  is more suitable for ionosphere monitoring over Brazil. This assessment would be useful for the research of the ionosphere and for GNSS-based applications. The four sources/providers of TEC maps employed here are the IGS (International GNSS Service), the Brazilian INPE/EMBRACE, the Argentinian UNLP-FCAGLP/MAGGIA, and the Japanese University of Nagoya. TEC maps employed here encompass the years from 2022 to 2024, in the current solar cycle 25 growing phase, according to their temporal and spatial availability, considering coverage of the Brazilian territory. Some months were chosen each year, March, June, September and December in order to represent seasonal behavior of the ionosphere.

The proposed comparison required a search for temporal and spatial gaps of the time series of maps of each source, which are due to lack of data, thus filtering the available maps. Each map is represented as a matrix that contains TEC point values for the considered [latitude x longitude] grid. In this text, such matrices are referred to as TEC map data. Even for the same source there are significant differences in the density of monitoring stations around the globe. A filtering concerning available time periods was done in order to assure overlapping periods, followed by an interpolation to obtain a unified spatial resolution. TEC map data of these four sources over Brazil are then compared by means of the Pearson correlation coefficient and the Structural Similarity Index Measure (SSIM) values between each pair of maps and between each pair of sources. Significant differences were found, i.e. low correlations between pairs of maps.

In order to present an example of how TEC maps could be potentially employed for detecting ionospheric bubble signatures, an example is also presented, comparing the sequence of three TEC maps to the corresponding maps of amplitude scintillation and ROTI (Rate of TEC Index) maps.

A foremost analysis is the validation with ground truth of the TEC maps of the four sources, in which TEC point values derived from a set of GNSS stations in Brazil are used as references.  It is worth noting that the point values extracted from the TEC maps are subjected to interpolation errors.

This work draws as a conclusion that none of the TEC maps provided by these four sources are reliable, due to one or more of the following reasons: poor low spatial and/or temporal resolution, low density of GNSS monitoring stations employed to generate the maps,  and poor inadequate interpolation scheme.

The proposed comparative analysis is particularly significant in regions where ionospheric irregularities, such as equatorial plasma bubbles, are prevalent and could drastically degrade GNSS signal quality [11]. Despite numerous TEC map products, few research works have performed a comprehensive comparative investigation over a southamerican country using homogeneous spatiotemporal standards and ground truth validation.

# 2. Materials and Methods

Comparing TEC maps of different sources that have different spatial and temporal resolution is a difficult task. IGS, Nagoya, EMBRACE and MAGGIA maps were compared for the selected months of the 2022-2024 period, which are March, June, September and December. These four TEC map sources are described in Section 2.1. The matrices composed by the set of TEC values for the spatial grid of the maps are denoted here as TEC map data files. All these data files were time-filtered and interpolated to an unified grid in order to allow direct comparisons between pairs of maps of different sources for each day/time considering the Brazilian territory and surrounding countries.

The flowchart of the adopted methodology is shown in Figure 1, being composed of the steps of data acquisition of available TEC maps, preprocessing, spatiotemporal pairing and comparative analysis. The step of data acquisition and preprocessing comprehends the download from Internet sites of the TEC map sources/providers and the storage of the  corresponding TEC values for each grid point in files with the convenient format. The spatiotemporal pairing, described in Section 2.2 (“Generation of TEC maps”), is achieved by following procedures shown in Figure 1: (i) temporal filtering that selects periods in which maps overlap in time; (ii) sampling for the same time resolution; (iii) mask application for the Brazilian territory; (iv) interpolation to a common grid. Procedures (i) and (ii) appear in Section 2.3.1 (“Matching of temporal coverage and temporal resolution”). Procedures (iii) and (iv) appear in Section 2.3.2 (“Interpolation to a common grid over Brazil”).

The temporal filtering procedure (i) allowed the generation of 3 different datasets related to the TEC maps, according to the chosen months or the chosen set of TEC map sources. These datasets, denoted as TFs (time-filtered), are TF-120 and TF-30, which have time resolutions of 120 and 30 minutes, respectively, and both  undergo the same remaining procedures (ii, iii, and iv). As a result, these 3 datasets contain TEC maps with an unified spatial and temporal resolution, enabling the comparative analysis shown in Section 2.4 (“Procedure for the comparison of TEC map data"). It is composed of the Pearson correlation calculation between pairs of maps, the analysis of bubble signatures in TEC maps for a particular event, and the validation of TEC point values extracted from the maps for a set of GNSS stations using values of the Gopi Seemala Application [12].

Figure 1. Flowchart of the adopted methodology, showing the component steps to obtain TEC maps with a unified spatial and temporal resolution for the comparative analysis related to datasets TF-120 and TF-30.

## 2.1. TEC Maps and the Earth’s Ionosphere

Earth’s ionosphere is affected by the space weather, which is related to the Earth's interaction with outer space, being the biggest effect due to the Sun. Solar activity influences Earth magnetic and electric fields, which influence the ionosphere, a layer of the atmosphere that ranges from 50 to 1,000 km of altitude composed of plasma, i.e. ions and free electrons. Plasma density varies with altitude since it depends on the intensity of the solar ionizing radiation, plasma absorption by atmospheric gases through ion recombination and electron-capture processes, ion recombination, and electron-capture processes. Dynamic processes like diffusion, plasma drifts and neutral winds also affect the plasma density profile. In the ionosphere there is a competition between ionization production due to the Sun radiation and ionization losses due to the recombination of ions and electrons. The ionospheric ionization is much larger during sunlight hours than nighttime hours when there is no more solar radiation. The ionosphere also is influenced by seasonality, geographic location, phase of the solar cycle, and magnetic storms [13,14].

Radio frequency signals between satellites and ground receivers are affected by the ionosphere, mainly by perturbations known as ionospheric scintillation, caused by irregularities of the ionospheric density. The most important of these radio signals are from Global Navigation Satellite Systems (GNSS), constellations of satellites that provide precise geographic position to receivers in aircrafts, ships, vehicles and ground stations. Once the GNSS receiver acquires a lock with at least four satellites, its position is calculated by triangulation from each estimated receiver-satellite distance. Currently operational GNSS are the American Global Positioning System (GPS), the Russian Global Navigation Satellite System (GLONASS), the Chinese BeiDou Navigation Satellite System (BDS), the European Galileo, and the Japanese Quasi-Zenith Satellite System (QZSS) [15].

Ionospheric irregularities span a wide range of spatial and temporal scales and can seriously affect the performance of satellite-based navigation systems such as GNSS. While TEC may be used to characterize large-scale ionospheric characteristics, small-scale plasma irregularities have the ability to produce signal phase and amplitude variations, known as scintillation. Classical work, for instance, [16], has provided the theoretical and experimental foundation to understand generation of irregularity, particularly at low-latitudes. Contemporary research, for instance, [17,18], has employed ground- and space-based GNSS measurements to outline the multi-scale nature of the irregularities. After decades of research, accurate monitoring and prediction of such phenomena remain difficult, especially within equatorial latitudes [19]. Some studies explore the potential of using remapped TEC images to capture ionosphere bubble signatures [20].

The TEC field is estimated from dual-frequency measurements of phase delay, or adding the pseudorange, made by receivers of networks of ground stations. Worldwide or regional TEC maps are obtained by interpolating local TEC values derived from measurements acquired by these receivers. Mathematical models are also used to generate the TEC field and the corresponding maps, but usually are not accurate due to low temporal or spatial resolutions, and lack of assimilation of data from GNSS stations.

During daylight the equatorial ionospheric plasma is lifted up due to eastward electric fields. After sunset and until about 21 LT (local time) this upward plasma movement is intensified, giving origin to a large vertical electron density gradient at the ionospheric F region base, which is one condition for plasma irregularities to be generated through the Rayleigh-Taylor mechanism [21].

One ionospheric anomaly is the Equatorial Ionization Anomaly (EIA), which occurs about 15° magnetic degrees north and south of the dip equator, characterized by larger background electronic density during the pre-reversal hours (18-21 LT) compared to lower latitudes [22]. The TEC field at EIA shows crests with high gradients, which are associated with the eventual occurrence of strong scintillation.

The most common of the ionospheric irregularities are the ionospheric bubbles, low-density structures (low TEC values) that are formed near the magnetic equator or at low magnetic latitudes, mostly in the equinoxes and summer of years of high solar maximum activity. These bubbles are created 1-to-2 hours after sunset, expand along the magnetic north and south directions, then migrate to the magnetic south and west directions, and disappear before midnight and sometimes 1-to-2 hours after midnight. Inside the equatorial plasma bubbles, small-scale ionospheric plasma irregularities are generated, causing amplitude and phase scintillation which affects RF communication in the L band, for instance GNSS navigation. Ionospheric scintillation is measured by the amplitude and phase indexes. Amplitude scintillation is given by the S4 index, calculated as the normalized standard deviation of the GNSS signal intensity for 1 minute with a sampling rate of 50 Hz [23]. Phase scintillation is given by the σφ index, calculated as the standard deviation of the detrended phase in each 1-minute interval [23,24]. Scintillation typically occurs in the months of September to March in Brazil.

Besides the increased frequency of bubbles in the equinoxes and summer, there is a high variability throughout the year, making it difficult to predict their occurrence. TEC gradients in a bubble can reach 30-50 TEC units (TECU) for some hundreds of kilometers [1,2]. Similarly to scintillation, these bubbles occur in general between the months of September and March in Brazil, presenting high TEC gradients and affect the ionosphere refraction index. They may cause scintillation, disturbing GNSS and telecommunication signals in amplitude and phase [25].

## 2.2. TEC maps sources

The ionosphere can be modeled by the TEC field, depicted by latitude-longitude maps. Values of TEC in these maps are obtained assuming the ionosphere as a thin layer at a defined altitude. Slant TEC (STEC) is defined as the line integral of the electron density (ne) given by the line integral along the signal slant path s from receiver to satellite, considering a column of one-meter-square cross section centered on the path, which is given in TEC units, defined as 1 TECU = 1016 electrons/m², as shown below:

| STEC= ne ds | (1) |
| --- | --- |

Typical TEC values measured near the Earth’s surface range from about 1 to 150 TECU [26] with the actual value depending on geographic location, local time, season, solar flux, and magnetic activity. A relative STEC can be estimated by observing carrier phase and pseudorange of the received RF signals recorded by multi-frequency GNSS receivers. After leveling STEC values derived from the carrier phase using those derived from the pseudorange, these STEC values are corrected for the ground receiver and the satellite biases to obtain the absolute STEC values. Finally, the vertical projection of these values results in the vertical TEC values (VTEC), here referred to as TEC.

Better TEC maps are derived using higher temporal and spatial resolutions, but global maps may have accuracy compromised by TEC data provided by low density of TEC monitoring stations according to the region. In the case of TEC map prediction, a long time series may be required, and the extension of the time series varies widely for different sources. TEC map prediction using machine learning requires a long time series in order to perform the training of a neural network (most common case). Another point is to have a time series covering entire solar cycles, not only solar maximum or solar minimum. However, this study covers the current growing phase of the solar cycle 25. Two global and two regional TEC map sources are analyzed here. Table 1 describes these four TEC map sources.

Table 1. TEC maps sources of this study.

| Source | Spatial resolution (lat x long) | Temporal resolution (min) | Spatial coverage | TEC map extension |
| --- | --- | --- | --- | --- |
| IGS | 2.5° x 5° | 120 | Global | (87.5° S to 87.5° N; 180.0° W to 180.0° E) |
| INPE/EMBRACE | 2° x 2.5° | 10 | South America | (60.0° S to 20.0° N; 90.0° W to 30.0° W) |
| MAGGIA | 0.5° x 0.5° | 15 (until 09/04/2024) 10 (since 09/05/2024) | Central and South America, the Caribbean and Antarctic Peninsula | (80.0° S to 40.0° N; 110.0° W to 0°) |
| University of Nagoya | 0.5° x 0.5° | 5 | Global | (89.9° S to 89.6° N; 180.0° W to 180.0° E) |

Detailed information about these four TEC map sources is available in Appendix A (Sections A1.1, A1.2, A1.3 and A1.4).

## 2.3 Generation of TEC maps

This study does not compare TEC maps of different sources directly, since there are differences in temporal and spatial resolutions, time and area coverage that would make such comparison very subjective. The rest of this section explains the steps employed to perform such comparisons.

An initial step is to download and process data from each source in the corresponding formats (for instance, IONEX). The resulting processed data are TEC map matrices, one for each map, in the related original grid resolution. Each value of the matrix refers to a specific latitude and longitude. Outliers and negative TEC values were not treated, since they are very scarce, less than 1% in the worst cases. On the other hand, NaN (not-a-number) values are identified if not already indicated as such. The percentage of TEC NaNs in the original maps surpassed 50% according to the map, due to incomplete coverage or unavailability of data. This issue can be seen, for instance, in Figure A1 in Appendix B, which presents original TEC maps for the 2024 Solstices and Equinoxes. However, after the temporal filtering and interpolation to an unified grid, the remaining NaNs of the original maps of grid points were replaced by zero TEC values, as can be seen in the corresponding maps that appear in Figure 3. Otherwise, NaNs that were surrounded by valid TEC values were interpolated using neighboring grid points As a result, not any NaN value persisted in the interpolated/filtered grid point values of datasets TF-120 and TF-30.

Each original TEC map matrix is then processed in order to obtain the corresponding new TEC map matrix with unified temporal and spatial resolutions and for nearly-identical areas covering Brazil. This allows for a direct comparison between matrices of TEC values of different sources for the same time. This section details the main steps that were required.

### 2.3.1. Matching of temporal coverage and temporal resolution

The limited availability of TEC maps from the four sources for the selected four months of March, June, September and December of years 2022, 2023 and 2024, has implied in using maps of only three of the selected months for 2022 (all except December) , none in 2023, and maps of all four of the selected months for 2024. Even for the months with available maps, some days were not available. The complete description of the available maps of the four sources for days/months is available in Appendix A (Table A1). These months (except for June) were chosen as presenting more or increasing ionospheric irregularities and thus more ionospheric scintillation, since better TEC maps are expected to show such irregularities that correspond to high TEC gradients.

In order to cope with the different temporal coverages between the TEC map sources, two derived datasets, denoted as time-filtered (TF) datasets, were generated from the different TEC map sources, according to the following filtering criteria. Data corresponds to matrices of TEC values for the map grid. Two specific times of day time (08:00 and 16:00 UT) were selected to check the increasing TEC values along the day and a specific period of 20:00 to 04:00 UT was chosen as being more likely to present ionospheric scintillation, which is associated to high TEC gradients and thus high TEC values.

- Dataset TF-120: Data intersection for 120-minute resolution, considering only complete data between the four TEC map sources for the months (7) of March/2022, June/2022, September/2022, March/2024, June/2024, September/2024 and December/2024, and for the times of day 08:00, 16:00, 20:00, 22:00, 00:00, 02:00 and 04:00 UT (7 times of day), resulting in 183 days of valid data, totalizing 183 x 7, or 1,281 maps overall for each of the four sources.

- Dataset TF-30: Data intersection for 30-minute resolution, considering only complete data between the MAGGIA, Nagoya and EMBRACE TEC map sources, which have higher temporal resolution than IGS, for the months (4) March/2024, June/2024, September/2024 and December/2024, and for the times of day 08:00, 16:00, 20:00, 20:30, 21:00, 21:30, 22:00, 22:30, 23:00, 23:30, 00:00, 00:30, 01:00, 01:30, 02:00, 02:30, 03:00, 03:30 and 04:00 UT (19 times of day), totalizing 106 x 19, or overall 2,014 maps overall for each of the three sources.

### 2.3.2. Interpolation to a common grid over Brazil

The TEC map matrices described above are used to generate the corresponding new TEC map matrices with unified spatial resolution of 1° x 1°. A common area covering Brazil and part of some neighboring countries was defined as 39° S to 9° N, and 78° W to 30° W, requiring to crop each map to fit this area. The resulting unified TEC map matrix for each map has 49 x 49 elements.

This was achieved by mapping the grid of each original TEC map matrix of the original grid to the grid values of the unified grid. Such mapping is performed using a standard interpolation technique, interpolation by Inverse Distance Weighting (IDW), adjusting the different spatial resolutions of the different sources, some coarser and some finer, into the unified 1° x 1° resolution in latitude and longitude. In order to perform a fair interpolation at the borders, IGS and Nagoya data were adjusted to the original MAGGIA area coverage.

The IDW interpolation computes the value of each unified grid point taking into account the values of its neighboring original grid points, defined as the grid points that fall inside a given radius of influence.

The choice of IDW with radius of influence allows for a smooth interpolation, taking into account TEC values of neighboring grid points, and not propagating TEC values in neighboring areas with no valid grid points, nor creating image artifacts. This is the case of areas over the Amazonian jungle and areas far from the Brazilian coast, mainly the northeast coast.

IDW is a deterministic interpolation method suitable for interpolating a scattered set of input points. Points without known values can be assigned with a weighted average of the known point values. This interpolator yields the value u at a given point x based in the samples ui = u(xi) for i = 1, 2, …, N using the IDW function below:

| ux=i=1Nwi(x) uii=1Nwi (x) | (3) |
| --- | --- |

The above equation applies if d(x, xi) ≠ 0 for all i. Otherwise, u(x) returns ui itself (a point with known value). In the same equation, the term wi (x) is defined by:

| wi(x)=1dp(x, xi) | (4) |
| --- | --- |

In the above equation, x is the point to be interpolated, xi is a known point, d is the distance from every known point to the point to be interpolated, N is the number of known points, and p is a real positive number called power factor. In short, the value of every interpolated point is estimated from the values of all known points. An influence radius R defines the neighboring points that will be considered. It can act as a smoothing factor. This is done by modifying the function wk (x) as follows:

| wk(x) = max(0, R-d(x, xk))R d(x, xk)2 | (5) |
| --- | --- |

The distance metrics employed in the IDW interpolation was the Haversine. Even considering the Earth as a perfect sphere, distances along a parallel shorten as latitude increases, going away from the Equator. The Haversine distance, or great circle distance is defined to calculate the distance d(x,y) between two points x and y at the surface of a sphere, with latitudes x1 and y1 and longitudes x2 and y2, given in radians:

| D(x, y)=2arcsinsin2((x1-y1)/2)+cos(x1)cos(y1)sin2((x2-y2)/2) | (6) |
| --- | --- |

## 2.4. Procedure for the comparison of TEC maps

In this work, the metrics chosen for comparing pairs of TEC map data are the Pearson correlation coefficient (ρ), and the Structural Similarity Index Measure (SSIM). As previously mentioned, the proposed comparison between TEC maps of different sources by means of correlation is performed comparing the corresponding TEC map matrices adjusted for an unified temporal and spatial resolutions, and for the same coverage over Brazil and part of its neighboring countries. Each unified TEC map matrix consists of TEC grid values for the 1° x 1° grid with 49 x 49 elements. The SSIM comparison is performed using the raster images derived from these TEC maps. The results of the comparisons using the Pearson correlation coefficient and the SSIM are shown in Section 3. The datasets TF-120 and TF-30, used in the comparisons, were described in Section 2.3.1.

The first metric employed to compare TEC maps is the Pearson correlation coefficient [27], a well-known statistical measure that evaluates the strength of a linear relationship between each pair. It is given by the ratio of the covariance of the pair of TEC map data to the product of their standard deviations, resulting in a normalized value that ranges from −1 to +1. The value of +1 corresponds to a perfect linear correlation, while -1 indicates a perfect anticorrelation, and a value of zero is inconclusive, suggesting no linear relationship. Given a pair of matrices (X, Y) corresponding to TEC map data, the Pearson correlation coefficient ρ is given by:

| ρX, Y=covX,YσX* σY | (7) |
| --- | --- |

Above, cov is the covariance, and σX is the standard deviation of matrix X and σY, of matrix Y. Each pair of matrices, i.e. each pair of TEC map data, yields a Pearson correlation value. However, it is not suitable to calculate the joint correlation of hundreds of matrices by simply calculating the average of the corresponding correlations, since they usually do not follow a normal probability distribution function. If a joint correlation with a confidence interval is desired, as in the current work, the Fisher transformation [28,29] is employed to obtain a Fisher correlation coefficient z for each Pearson value ρ using the inverse hyperbolic arctangent function, also known as the Fisher’s transformation:

| z=12 ln 1+ ρ1- ρ | (8) |
| --- | --- |

The set of Fisher correlation coefficients follows a less skewed distribution and it is less susceptible to distributions with a small number of samples than the set of Pearson correlation coefficients. The application of the Fisher transformation is then followed by its inverse transformation to obtain a joint Pearson correlation value with, for instance, a 95% confidence interval, as in this work.

The second metric employed to compare TEC maps is the SSIM [30]. The comparison using the Pearson correlation is calculated using pairs of matrices corresponding to the maps. However, the SSIM is calculated using pairs of raster images, and thus the TEC maps to be compared were rasterized. SSIM is a standard technique originally developed to assess the quality of images by comparing a degraded image to the original one, but it was further extended as a metric to evaluate the similarity between two images considering structural information such as luminance, contrast, and spatial relationships. The SSIM is computed as follows, for the set of pixels of raster images X and Y:

SSIMX,Y = [(2μXμY + C1) * (2σXY + C2)][(μ2X + μ2Y + C1) * (σ2X + σ2Y + C2)] , where:  (9)

- μX and μY are the mean values of the pixels of X and Y, respectively;

- σ²X and σ²Y are the variances of X and Y, respectively;

- σXY is the covariance between X and Y, respectively;

- C1 e C2 are small constants introduced to stabilize the computation when denominators are close to zero; these constants are defined in Equation 10, where K₁ and K₂ are algorithm parameters (default values of 0.01 and 0.03, respectively), and  D is the dynamic range of the pixel values.

C₁ = (K₁ * D)² and C₂=(K₂ * D)²		    (10)

SSIM values range from -1 to 1, with a value of 1 indicating a perfect similarity between the compared TEC images. In cases where either image exhibits negligible variance, as in the case of uniform or near-uniform TEC images, the SSIM code adjusts its computation by comparing the mean absolute difference normalized by the dynamic range of the data. A joint SSIM value combining many SSIM values is also obtained by applying the Fisher transform and its inverse. SSIM provides an essential complement to the Pearson correlation coefficient, focusing on structural fidelity rather than just the linear relationships of the correlation. Thus, the use of the both metrics, Pearson correlation coefficient and SSIM, allows a comprehensive comparison of TEC maps.

## 2.5. Procedure for the validation of TEC maps with local values

The validation of TEC maps shown in Section 3.4 compares local TEC values derived from a set of GNSS stations, assumed as ground truth, with the corresponding local values extracted from the maps. Besides the statistical metrics described in the former section, MAE, RMSE and Pearson correlation, this validation employs two complementary statistical metrics, the Taylor Skill Score (TSS) and the Kling-Gupta Efficiency (KGE). In the definitions shown below, estimated values refer to TEC values extracted from the maps, while true values, to TEC values given by GNSS stations.

The Taylor Skill Score (TSS) is a metric derived from the Taylor Diagram that simultaneously evaluates the correlation and relative variability between estimated and true values [31]. The TSS ranges from 0 to 1, where 1 represents perfect agreement, and is defined as shown below:

TSS =   4 1 + ρ(σf + 1 / σf)² (1 + ρ0)   , where:          (11)

- ρ is the Pearson correlation coefficient between estimated and true values;

- σf represents the normalized standard deviation given by the ratio between the standard deviations of estimated and true values;

- ρ0 is a reference correlation value, often defined as the unity.

The Kling-Gupta Efficiency [32] represents a multi-component objective function developed for hydrological modeling, indicating goodness-of-fit between modeled and observed values. It evaluates three distinct components between modeled and observed values: correlation, variability bias, and mean bias. The adopted KGE formulation, applied here for estimated and true values, is given by:

KGE = 1-(ρ-1)²+(α-1)²+(β-1)² , where:     (12)

- ρ is the Pearson correlation coefficient between modeled and true values;

- α = σp /σo is the variability ratio (ratio of standard deviations);

- β = μp/μo is the bias ratio (ratio of means);

- σp and μp are the standard deviation and mean of modeled values;

- σo and μo are the standard deviation and mean of true values.

KGE values range from unity to negative values. A perfect KGE has unity value, indicating ideal performance across all three components (ρ=1, α=1, β=1). Decreasing, less than unity, KGE values indicate deteriorating performance. In the context of modeling, negative values suggest that the mean of observations is a better estimator than the model.

# 3. Results

## 3.1. Correlation and Similarity Results

In all presented results, “correlation” always refers to the Pearson correlation coefficient, and “similarity”, to the Structural Similarity Index Measure (SSIM). Correlations are calculated for each pair of matrices, i.e. each pair of TEC map data, and for all sources of TEC data, while SSIM is calculated analogously, but for the corresponding raster images of the TEC maps. The following sections present results using the TF-120 and TF-30 datasets. In the latter, correlations considering TEC values equal or above the third quartile (Q3) of the considered TEC values are also shown. In each comparison, the set of resulting correlations is reduced to a single correlation value using the Fisher transformation and its inverse.

### 3.1.1. Correlation results using the TF-120 dataset

Correlations are calculated between pairs of TEC map data with unified spatial and temporal resolution, i.e. between the corresponding matrices with TEC grid values for each time. All four sources are considered and time resolution is 120 minutes. Using the TF-120 dataset, comparisons are performed for a given source with all other sources, and also for a given source with each one of the other sources. In the first case, there are 3,843 comparisons between pairs of maps, while in the second case, 1/3 of that value, i.e. 1,281 comparisons. The latter is broken down by month as: 182 for 03/2022, 09/2022, 03/2024 and 06/2024; 175 for 06/2022 and 09/2024; and 203 for 12/2024.

In the first case, the joint correlation for the 3,843 comparisons between pairs of maps of each source with the remaining three sources using TF-120 is:

- MAGGIA x All other sources:      0.80;

- IGS x All other sources: 	    0.79;

- EMBRACE x All other sources:    0.64;

- Nagoya x All other sources:          0.63.

In the second case, the complete set of joint correlations for each of the 1,281 comparisons of each source with the other three, by month and for all months using TF-120, is available in Appendix B (Table A2).

### 3.1.2. Similarity results using the TF-120 dataset

SSIM values are calculated between pairs of raster images derived from the TEC map data with unified spatial and temporal resolution. All four sources are considered and time resolution is 120 minutes. Using the TF-120 dataset, SSIM comparisons are performed for a given source with all other sources, and also for a given source with each one of the other sources. In the first case, there are 3,843 comparisons between pairs of maps, while in the second case, 1/3 of that value, i.e. 1,281 comparisons. The latter is broken down by month as: 182 for 03/2022, 09/2022, 03/2024 and 06/2024; 175 for 06/2022 and 09/2024; and 203 for 12/2024.

In the first case, the joint SSIM value for the 3,843 comparisons between pairs of maps of each source with the remaining three sources using TF-120 is:

- MAGGIA x All other sources:  	   0.82;

- IGS x All other sources: 	   0.81;

- EMBRACE x All other sources:   0.79;

- Nagoya x All other sources:         0.75.

In the second case, the complete set of joint SSIM values for each of the 1,281 comparisons of each source with the other three, by month and for all months using TF-120, is available in Appendix B (Table A3).

### 3.1.3. Correlation results using the TF-30 dataset for the 3rd quartile

Correlations are calculated between pairs of TEC map data with unified spatial and temporal resolution using the TF-30 dataset, but comparisons only apply for TEC values equal or above the 3rd quartile (Q3) of each TEC source. All sources, except IGS are considered and time resolution is 30 minutes. Thus, considering a given map of the considered TEC source, only the set of TEC grid values of that map equal or above Q3 is used for the calculation of the correlation, which also uses the corresponding set of the grid values of the other source. In the first case, there are 4,028 comparisons between pairs of maps, while in the second case, half of that value, i.e. 2,014 comparisons. The latter is broken down by month as: 494 for 03/2024 and 06/2024; 475 for 09/2024; and 551 for 12/2024.

In the first case, the joint correlation for the 4,028 comparisons between pairs of maps of each source with the remaining two sources considering only TEC values above or equal Q3 using TF-30 is:

- EMBRACE x All other sources:   0.30;

- Nagoya x All other sources:  	   0.22;

- MAGGIA x All other sources:     0.17.

Also for the first case, but for the full range of TEC values, the joint correlation for the 4,028 comparisons between pairs of maps of each source with the remaining two sources using TF-30 is:

- MAGGIA x All other sources:     0.61;

- EMBRACE x All other sources:   0.60;

- Nagoya x All other sources:  	   0.60.

In the second case, the complete set of joint correlations for each of the 2,014 comparisons of each source with the other two, by month and for all months using TF-30, is available in Appendix B (Table A4). Please note that Q3 is different for each source, and correlations are calculated considering the first source as reference. Therefore, for instance, correlation between MAGGIA x EMBRACE is different from EMBRACE x MAGGIA, thus yielding two values for each pair of sources.

Also, considering TEC values above or equal Q3, the correlation values by month comprises 988 comparisons for 03/2024 and 06/2024, 950 for 09/2024 and 1,102 for 12/2024 (total of 4,028), shown in Table 2. In addition, the same table shows joint correlations considering the full range of TEC values.

Table 2. Joint correlations by month of pairs of TEC map sources using TF-30 for the full range of TEC values and for TEC values equal or above Q3, shown as the first and second values in each cell, respectively (best value is marked in blue, and the worst, in red).

| Source | All months | 03/2024 | 06/2024 | 09/2024 | 12/2024 |
| --- | --- | --- | --- | --- | --- |
| EMBRACE x All | 0.60 / 0.30 | 0.64 / 0.29 | 0.73 / 0.36 | 0.56 / 0.25 | 0.46 / 0.27 |
| MAGGIA x All | 0.61 / 0.17 | 0.67 / 0.18 | 0.72 / 0.21 | 0.59 / 0.10 | 0.43 / 0.14 |
| Nagoya x All | 0.60 / 0.22 | 0.62 / 0.23 | 0.72 / 0.31 | 0.54 / 0.15 | 0.47 / 0.17 |

### 3.1.4. SSIM results using the TF-30 dataset

Similarity comparisons using maps with TEC maps equal or above Q3 would not be possible, since the SSIM calculation requires rectangular raster images corresponding to the same number of grid points. Thus, this section shows similarity results for the TF-30 dataset using maps with the full range of TEC values. As before, SSIM values are calculated between pairs of raster images derived from the TEC map data with unified spatial and temporal resolution. All four sources are considered and time resolution is 30 minutes. Using the TF-30 dataset, SSIM comparisons are performed for a given source with all other sources, and also for a given source with each one of the other sources. In the first case, there are 4,028 comparisons between pairs of maps, while in the second case, half of that value, i.e. 2,014 comparisons. The latter is broken down by month as: 494 for 03/2024 and 06/2024; 475 for 09/2024; and 551 for 12/2024.

In the first case, considering the full range of TEC values, the joint SSIM value for the 4,028 comparisons between pairs of raster images of each source with the remaining two sources using TF-30 is:

- EMBRACE x All other sources:  0.71;

- MAGGIA x All other sources:  	  0.68;

- Nagoya x All other sources:        0.67.

In the second case, considering the full range of TEC values, the complete set of joint SSIM values for each of the 2,014 comparisons between pairs of raster images of each source with the other two, by month and for all months using TF-30, is available in Appendix B (Table A5).

## 3.2. Case Study of Ionospheric Bubble Signatures on TEC maps

As previously mentioned, this case study is presented as an example of how TEC maps can potentially be used to identify ionospheric bubble signatures. Due to the availability of maps for this test case, only EMBRACE and MAGGIA maps are employed, and their high temporal resolution allows the use of 10-minute resolution.

Ionospheric bubbles are large ionization depleted structures that are generated at the magnetic equator. While drifting up at the magnetic equator they extend north and south to larger latitudinal sectors, along the magnetic field lines. As TEC maps cover a wide range of latitudes and longitudes, they allow the identification of the bubbles signatures as rarefied TEC regions. The bubble zonal movement is normally eastward during magnetically quiet days, which can be observed at TEC maps using a temporal sequence of images. Although TEC reflects typically large-scale ionospheric structure predominantly, its rate of change, calculated from the Rate of TEC Index (ROTI), has proved to be useful in measuring medium- to small-scale irregularities. It has been recently demonstrated that greater values of ROTI reveal strong correlation with the occurrence of ionospheric bubbles [33,34].

Inside plasma bubbles smaller scale size irregularities are generated causing scintillation in GNSS signals. Therefore, ROTI maps, and also, amplitude scintillation maps (S4 maps) show regions with moderate/strong scintillation (S4 > 0.5, and ROTI > 0.14 TECU/s), that may be associated with bubbles. In order to check if the TEC maps from different sources allow identifying bubble signatures, the corresponding ROTI and S4 maps were generated. It is worth mentioning that these maps are more accurate, present less artifacts and provide better coverage in regions with low density of GNSS monitoring stations like northern Brazil than the existing ROTI and S4 maps [35]. Interpolation errors were negligible in regions with denser coverage of stations based on validation tests using a set of specific stations.

A case study of ionospheric bubble signatures was chosen after a thorough search of scintillation occurrences in ROTI and S4 maps for the months of this study, and the scan of the corresponding TEC maps provided by EMBRACE and MAGGIA, searching for bubbles extending to the south. A single occurrence of scintillation matching bubbles was selected, which can be observed in a sequence of EMBRACE TEC maps, on September 27th, 2024 at 00:50, 01:00 and 01:10 UT over the Brazilian territory. These bubbles can not be clearly observed in the corresponding MAGGIA maps, despite their better 0.5° x 0.5° resolution in comparison with the coarser spatial resolution of 2.0° x 2.5° of the EMBRACE maps. Nagoya TEC maps were not considered due to lack of data on this date, nor IGS TEC maps, due to insufficient temporal resolution.

The value of the closest observation for the case study of the solar radio flux (F10.7 index), taken at September 26th, 2024, at 23:00 UT in Kaleden, Canada, was 182.7 solar flux units (1 sfu = 10−22 W.m−2.Hz−1). Considering the entire period adopted in this study, from March 1st, 2022, to December 30th, 2024, the initial value of F10.7 was 104.0 sfu, the final value was 222.8, and the average value, 161.0, always for 23:00 UT. Despite increasing solar activity in the period, clear signatures of ionospheric bubbles were observed in September, but not in December of 2024, considering the available maps.

Figure 2 shows the TEC, S4 and ROTI sequence of three maps for the September 27th (2024) test case of bubble occurrence. In this Figure, TEC maps are the original ones, adopting a TEC scale of 0 to 80 TECU for EMBRACE and 0 to 160 TECU for MAGGIA as shown in the vertical color bars, since the latter overestimates TEC values. The EMBRACE maps, shown in the 1st row of the Figure, allow observations of large TEC depleted structures, extending from the magnetic equator to about 20° S of dip latitude, but not allow a clear identification of  the bubbles. Larger TEC values are confined between 10° to 20° S, which is the EIA south crest latitudinal range for this period. Similar bubble signatures were presented by [2] using the EMBRACE TEC maps for February 15-16, 2014, with an enhanced resolution of 0.5° x 0.5°, but under demand. The 2nd row of Figure 2 shows the corresponding MAGGIA TEC maps, which are much smoother, precluding even more a clear identification of bubble signatures [20]. The 3rd row of the same Figure shows the corresponding maps for the ionospheric scintillation index S4 that is the normalized standard deviation of the amplitude scintillation in the L1 frequency. Scintillation is caused by the ionospheric plasma irregularities generated inside the bubbles. Finally, the 4th row of Figure 2 shows the corresponding maps for ROTI. It required the L1 and L2 frequencies to be calculated, using phase observables with 1-second resolution, with ROTI values being calculated every minute. It is worth to note that phase scintillation affects more ROTI maps than amplitude scintillation, and the ROTI were generated using a relatively dense subset of of 51 GNSS stations of the RBMC (Brazilian Network for Continuous Monitoring) monitoring network, provided by the IBGE (Brazilian Institute of Geography and Statistics). On the contrary, the S4 maps were generated using 30 GNSS stations from the INCT GNSS-NavAer network.

These S4 and ROTI maps show that areas with higher values correspond to the depleted regions of the EMBRACE TEC maps presented for the case study, providing evidence that TEC maps can be used to point out scintillation regions. This is very useful, since GNSS users can identify areas prone to the occurrence of scintillation, since the number of GNSS stations monitoring scintillation over Brazil is much lower than the number of GNSS stations monitoring TEC, mainly in the north of Brazil [36]. The ROTI and S4 scintillation maps were generated using a recent methodology that employs the Gaussian Process Regression (GPR) for interpolation and a specific set of pre-processing options, which corresponds to the use of the vertical projection of S4 values reduced by the average value above the 3rd quartile [35,37]. This methodology was developed in the scope of the Brazilian multi-institutional project National Institutes of Science and Technology (INCT) “GNSS Technology for Supporting Aerial Navigation” (GNSS-NavAer) [38].

In Figure 2, it is possible to compare the depleted ionospheric zones (three maps on first row), i.e. low TEC values, with zones that present scintillation (three maps on 3rd and 4th rows). The best match is observed for the EMBRACE maps. On the other hand, despite its higher resolution, MAGGIA TEC maps (2nd row three maps) presented higher TEC values than the EMBRACE ones, leading to smoother images that prevent a direct comparison with the ROTI and S4 scintillation maps, even with MAGGIA maps adopting a TEC scale that has the double of the range of the EMBRACE scale.

## 3.3. Performance of TEC Maps for the 2024 Equinoxes and Solstices

This section presents TEC maps of the different sources in order to allow a visual and the correlation/similarity comparisons for the two Equinoxes and the two Solstices of 2024. Both the TEC maps with original and unified spatial and temporal resolutions are shown (original ones appear in the Appendix).

Figure 2. TEC maps on September 27th, 2024 at consecutive times of 00:50, 01:00 and 01:10 UT generated by EMBRACE (maps on 1st row) and MAGGIA (maps on 2nd row), and corresponding S4 scintillation maps (maps on 3rd row) and ROTI maps (maps on 4th row) generated to observe ionospheric bubble signatures. In all maps, the continuous black line denotes the magnetic equator and the dashed lines, the dip latitudes spaced by 10°. The bottom-right corner of EMBRACE and MAGGIA TEC maps denotes the [minimum,maximum] range of its TEC values  and the name of the source.

In the Southern Hemisphere, the Autumn and Spring 2024 Equinoxes occurred, respectively, on 03/20/2024 at 03:06 UT and 09/22/2024 at 12:43 UT, while the Winter and Summer 2024 Solstices occurred, respectively, on 06/20/2024 at 20:50 UT and 12/21/2024 at 09:20 UT. Due to the limited 2-hour temporal resolution of the IGS maps, the adopted TEC maps were those with the closer times of the same day. Another restriction was the unavailability of MAGGIA maps for  06/20/2024 forcing the adoption of maps of all sources for the next day. These maps, either the original or the unified/interpolated are denoted as:

- AE24  - near Autumn Equinox, on 03/20/2024 at 04:00 UT;

- WS24  - next day of Winter Solstice, on 06/21/2024 at 00:00 UT;

- SE24   - near Spring Equinox, on 09/22/2024 at 14:00 UT;

- SS24   - near Summer Solstice, on 12/21/2024 at 10:00 UT.

Figure 3 shows the unified versions (same spatial and temporal resolution) of the TEC maps AE24, WS24, SE24 and SS24, generated by EMBRACE, IGS, MAGGIA and Nagoya. The corresponding original TEC maps of each source are available in Appendix B (Figure A1). TEC values vary according to the season of the year and to the phase of the solar cycle, being also subjected to eventual variations during magnetic storms. A study of TEC seasonality is out of the scope of this study, since the focus here is to evaluate the accuracy of TEC maps of different sources. In the case of the above maps, TEC values above 50-60 TECU are only observed for SE24 as it is for 14 UT, while AE24 and WS24 are for night times, and SS24 for early morning.

### 3.3.1. Correlation Results for TEC Seasonability

Using the unified/interpolated AE24, WS24, SE24 and SS24, TEC maps, taken as Equinoxes and Solstices samples for 2024, generated by EMBRACE, IGS, MAGGIA and Nagoya, joint correlations were calculated for a given source with all other sources, and also for a given source with each one of the other sources.

In the first case, the joint correlations, between each map with the other three considering maps AE24, WS24, SE24 and SS24 of each source, are:

- MAGGIA x All other sources:  	  0.79;

- IGS x All other sources: 	  0.79;

- EMBRACE x All other sources:  0.63;

- Nagoya x All other sources:  	   0.52.

In the second case, the correlations for the comparisons between pairs of maps for each 4-map set (AE24, WS24, SE24, SS24) of all sources, are shown in Appendix B (Table A6). Joint correlations for each source with the other three are also presented.

### 3.3.2. Similarity Results for TEC Seasonability

Using the unified/interpolated AE24, WS24, SE24 and SS24, TEC maps, taken as Equinoxes and Solstices samples for 2024, generated by EMBRACE, IGS, MAGGIA and Nagoya, SSIM values were calculated for a given source with all other sources, and also for a given source with each one of the other sources.

In the first case, the joint SSIM values, between each map with the other three maps AE24, WS24, SE24 and SS24 of each source, are:

- MAGGIA x All other sources:  	  0.78;

- IGS x All other sources: 	  0.77;

- EMBRACE x All other sources:  0.75;

- Nagoya x All other sources:        0.71.

Figure 3. Unified/interpolated TEC maps AE24, WS24, SE24 and SS24, TEC maps (shown in each row from left to right), taken as Equinoxes and Solstices samples for 2024, generated by (from top to bottom) EMBRACE (1st row), IGS (2nd row), MAGGIA (3rd row) and Nagoya (4th row). In all maps, the continuous black line denotes the magnetic equator and the dashed lines, the dip latitudes spaced by 10°. The bottom-right corner of each map denotes the [minimum,maximum] range of its TEC values  and the name of the source.

In the second case, the SSIM values for the comparisons between pairs of maps for each 4-map set (AE24, WS24, SE24, SS24) of all sources, are shown in Appendix B (Table A7). Joint SSIM values for each source with the other three are also presented.

### 3.3.3. Third-quartile correlation Results for TEC Seasonability

Using the unified/interpolated AE24, WS24, SE24 and SS24, TEC maps, taken as Equinoxes and Solstices samples for 2024, generated by EMBRACE, IGS, MAGGIA and Nagoya, joint correlations using TEC values equal or above the third-quartile (Q3) were calculated for a given source with all other sources, and also for a given source with each one of the other sources.

In the first case, the joint correlations, between each map with the other three, considering maps AE24, WS24, SE24 and SS24 of each source, and only for TEC values equal or above Q3 are:

- Nagoya x All other sources:  	   0.43;

- MAGGIA x All other sources:  	   0.31;

- EMBRACE x All other sources:   0.30;

- IGS x All other sources:	   0.21.

In the second case, all correlations for each set of 4 comparisons of each source with any of the other three are shown in Appendix B (Table A8). Please note that Q3 is different for each source, and correlations are calculated considering the first source as reference. Therefore, for instance, the correlation between MAGGIA x EMBRACE is different from EMBRACE x MAGGIA, thus yielding two values for each pair of sources.

In addition, for TEC values equal or above Q3, joint correlations considering maps AE24, WS24, SE24 and SS24 of all sources, are shown in Table 3. The same table shows correlations considering the full range of TEC values.

Table 3. Joint correlations for the unified/interpolated AE24, WS24, SE24 and SS24 TEC maps, considering the full range of TEC values or values equal or above Q3, shown as the first and second values in each cell, respectively (best value is marked in blue, and the worst, in red).

| Source | All dates | AE24 | WS24 | SE24 | SS24 |
| --- | --- | --- | --- | --- | --- |
| EMBRACE x All | 0.63 / 0.30 | 0.84 / 0.56 | 0.52 / 0.20 | 0.44 / 0.07 | 0.73 / -0.05 |
| IGS x All | 0.79 / 0.21 | 0.84 / 0.18 | 0.98 / 0.97 | 0.43 / -0.27 | 0.48 / 0.59 |
| MAGGIA x All | 0.79 / 0.31 | 0.96 / 0.40 | 0.98 / 0.90 | 0.10 / -0.01 | 0.60 / -0.01 |
| Nagoya x All | 0.52 / 0.43 | 0.66 / 0.53 | 0.58 / 0.64 | 0.44 / 0.15 | 0.48 / 0.50 |

## 3.4 Validation of TEC Maps with Ground Truth

The validation of the TEC maps of the four sources is performed using the ground truth provided by the Gopi Seemala application, which  generates local TEC time series for specific GPS monitoring stations. GPS raw data is formatted into RINEX 2 and RINEX 3 files that serve as input to the GPS-TEC analysis program [12]. In this section, it was assumed that local TEC measurements such as those of Gopi could be taken as reference as being more accurate than point values extracted from any TEC maps, since the latter embeds interpolation errors. Among other possible packages for TEC extraction like ICPT_TEC or Ionolab_TEC, the choice of the Gopi Seemala application was based in previous studies related to the ionosphere over Brazil, which included validation with independent observations such as ionosondes, all-sky imagers, and scintillation monitors [20,37].

In order to perform a comparison with local values extracted from the different  sources of TEC maps, the Gopi Seemala application was employed to generate TEC time series from the RINEX 3 data of RBMC GNSS stations in Brazil, assumed as local reference values. A set of error and accuracy metrics was calculated for the corresponding TEC values extracted from the TEC maps of each source: Mean Average Error (MAE), Root Mean Square Error (RMSE), Pearson correlation coefficient (ρ), Taylor Skill Score (TSS), and Kling-Gupta Efficiency (KGE) [31,32].

Figure 4 shows the RBMC GNSS monitoring stations over Brazil, with a total of 147 that were operational as for 2024, denoting by black dots the 27 that were employed in the validation, and by blue dots, the remaining ones (geographical coordinates of these stations are available in a repository). The same figure shows ellipses that delimitate a cluster of stations over the Amazonia and over the Brazilian southeast, each one with 9 RBMC stations.

The validation was performed considering each pair of TEC values given by the Gopi Seemala reference value and the corresponding value extracted from the map, for all times of day and all days. Thus, the error and performance metrics  were calculated using all these pairs of values for each source. Two different validation cases were considered:

- Validation of EMBRACE and MAGGIA TEC maps using TEC data from a reduced set of 7 GNSS stations for the example of ionospheric bubble signature in the three TEC maps shown in Section 3.2. (September 27th, 2024 at 00:50, 01:00 and 01:10 UT). As already mentioned, Nagoya and IGS maps were not available for this case of study. The 7 employed RBMC stations are BRAZ, CUIB, NAUS, POAL, SALU, SAVO, and SJSP, which are distributed over Brazil. The resulting metrics appear in Table 4 and in the Appendix B (table A9).

- Validation of TEC maps of all four sources using TEC data from the enlarged set of 27 GNSS stations for all days of December 2024. The Amazonian cluster is composed of the following 9 RBMC stations: AMTE, SAGA, PAAR, ROJI, RIOB, BELE, BOAV, NAUS, and POVE. The 9 RBMC stations of the Brazilian southeast cluster are: PPTE, SJSP, CHPI, SPTU, UFPR, SPFR, SPBO, SJRP, and MGIN. The remaining 9 RBMC stations also employed in the validation are: BRAZ, BRFT, CUIB, IMPZ, MSGR, POAL, SALU, SAVO, TOPL. This analysis is detailed in the next section.

Figure 4. GNSS stations of the RBMC monitoring network over Brazil employed in the validation of the TEC maps. The black dots denote the 27 selected stations, and the blue dots, the remaining stations, while the ellipses denote the two clusters of stations, the Amazonian and the Brazilian southeastern (adapted from Ref. [39]).

Table 4. Overall values for the set of metrics calculated for the EMBRACE and MAGGIA TEC local values corresponding to the seven chosen GNSS stations, using Gopi Seemala values as a reference, for September 27th, 2024 at 00:50, 01:00 and 01:10 UT (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE |
| --- | --- | --- | --- | --- | --- |
| EMBRACE | 18.48 | 20.91 | 0.90 | 0.64 | 0.48 |
| MAGGIA | 23.11 | 25.60 | 0.97 | 0.96 | 0.49 |

The more extensive validation (B) was performed for all times of day, all days, and all stations. The resulting error and accuracy metrics for the considered sources are shown in Table 5. In addition, considering all Gopi Seemala TEC values for all times of day, all days of December 2024, and all the 27 stations, it was possible to define three ranges of values: (i) Q-lower, containing all values below the first quartile, including it, (ii) Q-inter, with all interquartile values, and (iii) Q-upper, with all values above the third quartile, including it. The specific metrics for these three ranges of values appear in Table 6.

Table 5.  Overall values for the set of metrics calculated using TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps for the 27 selected GNSS stations, using Gopi Seemala values as a reference, for all times of day and all days of December, 2024 (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE |
| --- | --- | --- | --- | --- | --- |
| EMBRACE | 9.83 | 13.85 | 0.84 | 0.91 | 0.77 |
| IGS | 16.34 | 18.18 | 0.93 | 0.96 | 0.55 |
| MAGGIA | 21.13 | 23.61 | 0.83 | 0.88 | 0.42 |
| Nagoya | 17.56 | 25.57 | 0.51 | 0.71 | 0.41 |

Table 6. Overall values for the set of metrics calculated using TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps for the 27 selected GNSS stations, using Gopi Seemala values as a reference, for all times of day and all days of December, 2024, discriminated by quartiles (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE |
| --- | --- | --- | --- | --- | --- |
| EMBRACE (Q-lower) | 6.57 | 8.58 | 0.37 | 0.34 | -0.64 |
| EMBRACE (Q-upper) | 8.66 | 13.33 | 0.28 | 0.35 | -0.47 |
| EMBRACE (Q-inter) | 11.59 | 15.14 | 0.66 | 0.68 | 0.31 |
| IGS(Q-lower) | 16.24 | 17.49 | 0.49 | 0.42 | -1.16 |
| IGS(Q-upper) | 13.85 | 15.12 | 0.52 | 0.73 | 0.42 |
| IGS(Q-inter) | 17.21 | 19.07 | 0.81 | 0.86 | 0.43 |
| MAGGIA(Q-lower) | 16.66 | 18.15 | 0.37 | 0.28 | -1.52 |
| MAGGIA(Q-upper) | 23.18 | 25.62 | 0.25 | 0.22 | -1.22 |
| MAGGIA(Q-inter) | 21.87 | 24.06 | 0.67 | 0.65 | 0.09 |
| Nagoya_(Q-lower) | 8.40 | 10.36 | 0.25 | 0.20 | -1.34 |
| Nagoya(Q-upper) | 26.27 | 37.88 | 0.02 | 0.06 | -3.61 |
| Nagoya(Q-inter) | 17.59 | 22.84 | 0.38 | 0.42 | -0.24 |

Considering the TEC values extracted from maps of each source and the Gopi Seemala values taken as references for the two nine-station clusters (Amazonian and Southeastern), and for all times of day and all days of December 2024, the resulting metrics are shown in  Table 7. Furthermore, similar results appear in Table A10, but shown by quartiles. Surprisingly, even having a low density of GNSS stations, performance of the TEC maps for the Amazonian cluster was better than for the Southeastern cluster. This issue can be explained by the usually lower TEC values over the Amazonia, due to the effect of the Equatorial Ionization Anomaly (EIA). As commented before, lower TEC  values tend to improve the adopted metrics.

Table 7.  Overall values for the set of metrics calculated using TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps for the 27 selected GNSS stations, using Gopi Seemala values as a reference, for all times of day and all days of December, 2024, discriminated by the two clusters of stations (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE | Cluster |
| --- | --- | --- | --- | --- | --- | --- |
| EMBRACE | 6.40 | 10.60 | 0.91 | 0.95 | 0.88 | Amazonian |
| EMBRACE | 14.56 | 17.61 | 0.89 | 0.92 | 0.53 | Southeastern |
| IGS | 13.25 | 14.33 | 0.97 | 0.99 | 0.64 | Amazonian |
| IGS | 21.79 | 23.14 | 0.92 | 0.96 | 0.35 | Southeastern |
| MAGGIA | 18.60 | 21.12 | 0.87 | 0.89 | 0.51 | Amazonian |
| MAGGIA | 24.26 | 26.39 | 0.79 | 0.85 | 0.28 | Southeastern |
| Nagoya | 19.84 | 30.26 | 0.49 | 0.74 | 0.32 | Amazonian |
| Nagoya | 16.39 | 20.86 | 0.70 | 0.78 | 0.46 | Southeastern |

A similar validation was performed considering the TEC values extracted from maps of each source and the Gopi Seemala values taken as references for the 14 GNSS stations employed by IGS over Brazil, which are part of the RBMC monitoring network, also for all times of day and all days of December 2024.  The resulting metrics are shown in  Table 8.

Table 8. Overall values for the set of metrics calculated using TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps for the 14 IGS stations, using Gopi Seemala values as a reference, for all times of day and all days of December, 2024 (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE |
| --- | --- | --- | --- | --- | --- |
| EMBRACE | 9.30 | 13.44 | 0.84 | 0.91 | 0.80 |
| IGS | 16.24 | 18.01 | 0.93 | 0.97 | 0.55 |
| MAGGIA | 20.90 | 23.44 | 0.84 | 0.88 | 0.43 |
| Nagoya | 16.95 | 25.13 | 0.52 | 0.73 | 0.44 |

Yet another validation appears in Table 9, which is similar to the validation that resulted in Table 5, considering all Gopi Seemala TEC values for all days of December 2024, and all the 27 stations, but only for 18 UT, which is typically the time of day presenting high TEC values. Furthermore, metrics were similarly calculated for 18 UT, but discriminating by the three quartiles, as shown in Table A11 (similar to Table 6, but for 18 UT only). TEC values are more intense in Brazil at 18-19 UT.

Table 9.  Overall values for the set of metrics calculated using TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps for the 27 selected GNSS stations, using Gopi Seemala values as a reference, for 18 UT of all days of December, 2024 (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE |
| --- | --- | --- | --- | --- | --- |
| EMBRACE | 9.96 | 14.30 | 0.20 | 0.55 | 0.12 |
| IGS | 17.06 | 18.89 | 0.47 | 0.65 | 0.34 |
| MAGGIA | 25.05 | 26.95 | 0.29 | 0.42 | -0.25 |
| Nagoya | 25.56 | 36.67 | -0.09 | 0.12 | -1.82 |

In order to assess the performance of stations according to its geographical position, also for the different sources, another validation was performed, considering all times of day and all days of December 2024, per station. Such calculation rendered a 27-element distribution of each metric per source. Table 10 shows the mean values and standard deviation for each metric, and for each of the four sources. The complete set of metrics for each station and for each source is also available It can be observed that standard deviations are relatively low for MAE, RMSE and KGE and very low for the correlation and TSS, demonstrating an even performance of the 27 stations for each source.

Coefficients of variation (standard deviation divided by the mean) were also calculated for each case, demonstrating a low variability of performance (error and accuracy metrics) along the stations for each source.

Table 10. Mean values, standard deviation and coefficient of variation for the distribution of each metric along the 27 selected GNSS stations, corresponding to the TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps, using Gopi Seemala values as a reference, for all times of day and days of December, 2024 (MAE, σMAE , RMSE and σRMSE values in TECU).

| Source | MAE | σMAE | CVMAE | RMSE | σRMSE | CVRMSE | CORR | σCORR | CVCORR | TSS | σTSS | KGE | σKGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EMBRACE | 9.98 | 4.24 | 0.42 | 13.48 | 3.91 | 0.29 | 0.90 | 0.04 | 0.05 | 0.93 | 0.03 | 0.72 | 0.18 |
| IGS | 16.52 | 5.09 | 0.31 | 17.67 | 5.16 | 0.29 | 0.95 | 0.03 | 0.04 | 0.97 | 0.02 | 0.53 | 0.18 |
| MAGGIA | 21.27 | 3.42 | 0.16 | 23.54 | 3.52 | 0.15 | 0.84 | 0.06 | 0.07 | 0.88 | 0.04 | 0.40 | 0.14 |
| Nagoya | 17.57 | 4.55 | 0.26 | 24.90 | 5.59 | 0.22 | 0.61 | 0.15 | 0.25 | 0.76 | 0.08 | 0.40 | 0.16 |

# 4. Discussion

In Section 3.1.1, joint correlation analysis was performed for each pair of TEC maps and each pair of sources. Comparisons of one source against all other ones for the entire TEC range using the TF-120 dataset of 120-minute resolution show that MAGGIA and IGS maps are better correlated to the other maps (around 80%), followed by EMBRACE and Nagoya (around 60%). Concerning image similarity, shown in Section 3.1.2, the same ranking applies, but differences are smoother, with joint SSIM values around 80%.

In Section 3.1.3, joint correlation analysis was performed for each pair of TEC maps and each pair of sources using the TF-30 dataset of 30-minute resolution, which excluded IGS due to its 120-minute resolution. Comparisons were broken down in two groups: using only TEC values equal/above the third quartile (Q3) or using the full range of TEC values. Joint correlation results for the full range were very similar for EMBRACE, MAGGIA and Nagoya (around 60%), but considering TEC values equal or above Q3, correlation was better for EMBRACE (30%) than for Nagoya (22%) and MAGGIA (17%). Joint correlations using the full TEC range tend to be higher since the predominance of low TEC values tightens the range of values. Comparisons involving Q3 are more important, since for high values of Q3, the dynamics of the ionosphere is associated with high TEC gradients. However, in this case, correlations tend to be lower due to the interpolation methods of maps of different sources, which cause a mismatch of map grid points with high TEC values between these maps. In addition, grid points with such high TEC values are scarce. Similarity comparisons were performed for TF-30 considering the full range of TEC values, resulting in values of joint SSIM around 70% (EMBRACE value was slightly better).

Section 3.2 presented a case of ionospheric bubble signature in a TEC map, which points out the necessity of TEC maps with better accuracy and spatial/temporal resolution to that end. The rationale was that ionospheric bubbles, which are depleted ionization regions of the ionosphere, are associated with ionospheric irregularities that present high TEC gradients detectable in ROTI maps, which  may cause scintillation. Therefore, it can be expected a correspondence between TEC maps and ROTI or amplitude scintillation maps for such regions. The case study was defined from an occurrence of ionospheric scintillation that occurred on September 27th 2024, detected in a sequence of three amplitude scintillation maps, at 00:50, 01:00 and 01:10 UT, as well as the ROTI maps for the same date and times. The analysis of the corresponding TEC maps shows the matching patterns that characterize the bubbles.

Section 3.3 shows the performance of the different TEC maps for the 2024 Equinoxes and Solstices, considering the 4 corresponding maps denoted as AE24, WS24, SE24 and SS24. Joint correlation results were similar to those of TF-120, 79% for MAGGIA and IGS, while EMBRACE and Nagoya were 63% and 52% respectively. On the other hand, joint SSIM values were very close (around 75%), with a slight disadvantage for EMBRACE and Nagoya, as also were for TF-120. However, as seen in Figure 3, these maps for MAGGIA and IGS are quite similar, and results in Table A6 show corresponding values of 90% or above for correlation, while similarity results are around 90%.

Finally, a validation of TEC map local values was performed using TEC data provided by the Gopi Seemala application for a set of selected GNSS stations over Brazil, as shown in Section 3.4. Gopi Seemala values are assumed as reference values. i.e. as true values. The employed metrics are MAE, RMSE, Pearson correlation, TSS and KGE, applied to each set of local values extracted from each TEC map source for specific times/dates. In the example of the bubble signature the performance of EMBRACE and MAGGIA maps were compared for September 27th, 2024 at 00:50, 01:00 and 01:10 UT). MAGGIA presented the worst MAE and RMSE values, but better correlation and TSS values (KGE values were almost identical). MAGGIA values correlated better than EMBRACE with Gopi TEC reference values, but systematically overestimated these values. As Table A9 details, for most of the GNSS stations, MAGGIA overestimated TEC values by 50% or more, while EMBRACE overestimated less or even underestimated. In the case of the SJSP, EMBRACE yielded an almost exact value.

A more extensive validation was performed using 27 GNSS stations over Brazil, also assuming Gopi Seemala values as references the ground truth, for all times of day and all days of December 2024. In general, performance of EMBRACE was slightly better, followed by IGS, MAGGIA and Nagoya (IGS presented the best correlation and TSS values). The same analysis, but discriminating quartiles, presented similar results, mainly for the interquartile range. As it would be expected, correlation, TSS and KGE values were quite low for the upper and lower quartiles. This is due, respectively, for the possibly high mismatch of grid points with high TEC values with higher values in the maps for the corresponding station values, and for some variability that is inherent to low TEC values. Two subsets of the 27-station set were defined, each one with 9 GNSS stations, the Amazonian and the Southeastern clusters. As already commented, the performance of the TEC maps for the Amazonian cluster of stations was better than for the Southeastern, despite the region of the first cluster having a lower density of GNSS stations. Such unexpected results could be explained by the lower TEC values over the Amazonia, which tend to improve the values of the metrics.

Additionally, even considering validation for only the 14 GNSS stations employed by the IGS provider, the metrics were similar to those obtained with the set of 27 GNSS stations, and also presented a similar ranking for the four sources. Another experiment, considering the full set of 27 GNSS stations for all days of December 2024, but only for 18 UTC resulted in another similar ranking, but correlation, TSS and KGE values were low as expected, since TEC values at 18 UTC are high. Furthermore, only a single time of day is considered, implying a reduced set of 27 TEC-values, instead of 3,888 values for instance, in the case of all times of the day considering the 10-minute resolution of EMBRACE.

A final assessment considered validation for all times of day and all days of December 2024, but discriminated by each of the 27 GNSS stations. This scheme resulted in 27 distributions for each metric, each one with a mean value, a standard deviation and a coefficient of variation. The values of the latter two parameters demonstrated a low variability among the stations concerning these error and accuracy metrics.

# 5. Conclusions

This work proposed a comparison of TEC maps of different sources, IGS, EMBRACE, MAGGIA and Nagoya. As already mentioned in the previous section, such comparison is challenging, due to the different spatial and temporal resolutions, poor incomplete coverage of data for some regions or times, and different density of GNSS stations over Brazil. The task of drawing conclusions is subjected to the assumption of possible causes of inaccuracies. Moreover, the direct comparison between pairs of maps, performed by means of different metrics, presented results that cannot be interpreted directly, since there is not an absolute TEC map to reference. It can be assumed that the source/provider of TEC maps that was more correlated to all the others is the best, but such a conclusion is different for the full range of TEC values and for the values related to the 3rd quartile, which are more relevant, when such value is high.

A validation of these TEC maps was performed using a time series of Gopi Seemala TEC point values using a set of 27 specific GNSS stations geographically distributed over Brazil. It was assumed that such comparison is a validation, considering that the Gopi Seemala TEC values can be taken as a reference. This assumption presumes that Gopi Seemala algorithm for calculating TEC is correct, and that carrier/signal measurements are not affected by noise or other errors.

The validation involved different experiments, which included validation of GNSS stations of an Amazonian or a Southeastern cluster, validation considering quartiles, and validation only for the 18 UTC time (likely TEC value peak time). Moreover, statistics of the metrics according to the stations show low variability of performance of the TEC maps of different sources along the stations. All validation resulted in similar ranking for the considered four sources, which is maybe a clue of the robustness of the analysis.

This study compared systematically the four sources of TEC maps, IGS, EMBRACE, MAGGIA, and Nagoya, over Brazil during the rising phase of solar cycle 25 (2022–2024). Correlations results between pairs of maps of different sources and the validation using Gopi Seemala data were presented. These results allow to identify substantial divergences between TEC maps of these sources, which are probably due to GNSS monitoring station coverage, TEC calculation and map interpolation method. Validation errors were estimated using the MAE, RMSE, TSS and KGE metrics. Our results can be summarized as follows:

- IGS maps have global coverage and perform reasonably despite having low regional density of GNSS stations and poor low spatial and temporal resolutions, which implied coarse interpolation, and thus smoother maps. A higher temporal resolution would enhance the quality of these maps, but it would be unfeasible due to the processing demands of global maps;

- EMBRACE maps presented lower validation errors (MAE and RMSE), but its standard available spatial resolution could be improved, as demonstrated in the case study of bubble signature presented here. Another point is its map generation that includes interpolation, but not extrapolation of TEC values, and thus many NaNs are left, according to the map. These NaNs occur despite using a dense network of GNSS stations;

- MAGGIA maps have the better spatial and temporal resolution (equal to Nagoya), but they consistently overestimated TEC, implying higher validation errors, despite also using a dense network of GNSS stations. Its maps they could not be used did not allow to clearly detect the bubble signature case study in the example presented here;

- Nagoya maps presented validation errors similar to IGS EMBRACE, but the worst values for correlation, TSS, and KGE, even having but have a global coverage and better spatial and temporal resolutions. Its maps presented a higher amount of NaNs, probably because they were conceived for the very dense network of GNSS stations over Japan.

As shown from the obtained results, this work highlights that all analyzed TEC maps over Brazil could be more reliable. According to the considered source, it can be concluded that spatial/time resolution, map interpolation method or density of GNSS monitoring network should be improved to provide better TEC maps. As the number of GNSS stations over Brazil is increasing, a future work would involve performing an extensive validation of these TEC data sources using more GNSS stations over Brazil and closer regions of neighboring countries. Better TEC maps would enable studies of the ionosphere, such as the identification of ionospheric bubbles of the test case presented here. Furthermore, better TEC maps would also improve the evaluation of impacts in GNSS-based services such as positioning, navigation, and ionospheric modelling.

Author Contributions:

Conceptualization, Stephan Stephany and André R. Martinon; Data curation, Marco A. Cintra, André R. Martinon and Patrícia M. Negreti; Formal analysis, Stephan Stephany, Lamartine Guimarães, Eurico de Paula, Alison Moraes and Jonas de Souza; Funding acquisition, Eurico de Paula, Alison Moraes and Jonas de Souza; Investigation, Stephan Stephany, Eurico de Paula, André R. Martinon, Alison Moraes and Jonas de Souza; Methodology, Stephan Stephany, Eurico de Paula and André R. Martinon; Project administration, Eurico de Paula; Resources, Marco A. Cintra, André R. Martinon and Patrícia M. Negreti; Software, Marco A. Cintra and André R. Martinon; Supervision, Stephan Stephany and Lamartine Guimarães; Validation, Stephan Stephany, Eurico de Paula, André R. Martinon, Alison Moraes and Jonas de Souza; Visualization, Marco A. Cintra; Writing – original draft, Marco A. Cintra and Stephan Stephany; Writing – review & editing, Marco A. Cintra, Stephan Stephany, Lamartine Guimarães, Eurico de Paula, André R. Martinon, Patrícia M. Negreti, Alison Moraes and Jonas de Souza.

Funding: authors take part and acknowledge the Brazilian multi-institutional project INCT “GNSS Technology for Supporting Aerial Navigation” (GNSS-NavAer), supported by grants Conselho Nacional de Desenvolvimento Científico e Tecnológico (CNPq) 465648/2014-2, Fundação de Amparo à Pesquisa do Estado de São Paulo (FAPESP) 2017/50115-0, and Coordenação de Aperfeiçoamento de Pessoal de Nível Superior (CAPES) 88887.137186/2017-00. Author Marco Antônio de Ulhôa Cintra thanks Federal Institute of Education, Science and Technology of São Paulo (IFSP) Câmpus Caraguatatuba for the paid work leave to attend his doctoral studies. Eurico R. de Paula acknowledges the support from CNPq under process numbers 302531/2019-0 and 306076/2024-1. André R. F. Martinon acknowledges the CNPq grants 116293/2024-1 (Postdoctoral Research) and 386214/2024-7 (Technological and Industrial Development DTI-A). This study was financed in part by the Coordenação de Aperfeiçoamento de Pessoal de Nível Superior - Brasil (CAPES) - Finance Code 001.

Data Availability Statement:

The dataset and computer code employed in the article are openly available, respectively, in Zenodo at https://doi.org/10.5281/zenodo.15453941, and in GitHub at https://github.com/marcocintra/Atmosphere/.

Acknowledgments: authors thank the National Laboratory for Scientific Computing (LNCC) (Ministry of Science, Technology and Innovation (MCTI)/LNCC, Brazil) for the use of the Santos Dumont supercomputer, as part of the project “Implementation of machine learning approaches that demand supercomputing for ionospheric scintillation prediction and meteorological forecast of severe convective events”. Authors thank the Brazilian Ministry of Science, Technology and Innovation, and Brazilian Space Agency. Authors thank, for data supplying: 1) TEC data: (a) IGS; (b) INPE/EMBRACE; (c) UNLP-FCAGLP/MAGGIA; (d) Univ. Nagoya/ISEE; (e) GNSS RINEX files for the GNSS-TEC processing are provided from many organizations; 2) F10.7 data: National Research Council Canada (CNRC)/NRCan; 3) Scintillation and ROTI maps: DIHPA/INPE.

Conflicts of Interest: the authors declare no conflicts of interest.

# Abbreviations

The following abbreviations are used in this manuscript:

| BDS | BeiDou Navigation Satellite System |
| --- | --- |
| BKG | Federal Agency for Cartography and Geodesy |
| CAPES | Coordenação de Aperfeiçoamento de Pessoal de Nível Superior |
| CAS CNPq | Chinese Academy of Sciences Conselho Nacional de Desenvolvimento Científico e Tecnológico |

CGCE	    Coordenação-Geral de Engenharia, Tecnologia e Ciência Espaciais
CGIP	    Coordenação-Geral de Infraestrutura e Pesquisas Aplicadas
CNRC	    National Research Council Canada
CODE	    Center for Orbit Determination in Europe
COPDT	    Coordenação de Pesquisa Aplicada e Desenvolvimento Tecnológico
DGFI-TUM     Deutsches Geodätisches Forschungsinstitut-Technical University of Munich 
DIHPA	    Divisão de Heliofísica, Ciências Planetárias e Aeronomia
DOY	    Day-of-the-year
dTEC	    Detrended TEC
DTI	    Technological and Industrial Development
EIA	    Equatorial Ionization Anomaly	
EMBRACE	    Studying and Monitoring of the Brazilian Space Weather
ESA	    European Space Agency
EUV	    Extreme Ultraviolet
FAPESP	     Fundação de Amparo à Pesquisa do Estado de São Paulo
FCAGLP	     Faculty of Astronomic and Geophysical Sciences of the La Plata National University
GIM	     Global Ionospheric Maps
GLONASS	     Global Navigation Satellite System
GNSS	     Global Navigation Satellite Systems
GNSS-NavAer Global Technology for Supporting Aerial Navigation
GPR	     Gaussian Process Regression
GPS	     Global Positioning System
IAAC	     Ionospheric Associate Analysis Centers
IBGE	     Brazilian Institute of Geography and Statistics
IDW	     Inverse Distance Weighting
IFSP	     Federal Institute of Education, Science and Technology of São Paulo
IGM	     Military Geographical Institute
IGN	     National Geographical Institute
IGN	     National Institute of Geographic and Forest Information (France)
IGP	     Geophysical Institute of Peru
IGS	     International GNSS Service
INCT	     National Institutes of Science and Technology
INPE	     National Institute for Space Research
ISEE	     Institute for Space-Earth Environment Research
ITA	     Instituto Tecnológico de Aeronáutica
JPL	     Jet Propulsion Laboratory
KGE	     Kling-Gupta Efficiency
LT		     Local time
LISN	     Low latitude Ionospheric Sensor Network
MAGGIA	     Meteorología espacial, Atmósfera terrestre, Geodesia, Geodinámica, diseño de Instrumental y Astrometría
NaN	     Not-a-number
NASA	     National Aeronautics Space Administration
netCDF	     Network Common Data Form
NRCan	     Natural Resources Canada
OPTIMAP           Operational Tool for Ionospheric Mapping And Prediction
PG-CAP	     Programa de Pós-Graduacão em Computação Aplicada
PG-CTE	     Programa de Pós-Graduação em Ciências e Tecnologias Espaciais
QZSS	     Quasi-Zenith Satellite System
Q3		     3rd quartile		
RAMSAC	     Argentine Continuous Satellite Monitoring Network
RBMC	     Brazilian Network for Continuous Monitoring
REGNA-ROU  Active National Geodesic Network-Eastern Republic of Uruguay
RIM	     Regional Ionospheric Maps
RINEX	     Receiver INdependent EXchange
ROTI	     Rate of TEC index
rTEC	     TEC difference ratio
SBAS	     Satellite-Based Augmentation System	 
sfu		     Solar flux unit
SJC	     São José dos Campos
SSIM	     Structural Similarity Index Measure
STEC	     Slant Total Electron Content
TEC	     Total Electron Content
TECU	     TEC units
TF		     Time-filtered
TSS	     Taylor Skill Score 
Unitau	     Universidade de Taubaté
UNLP	     La Plata National University
UPC	     Universitat Politècnica de Catalunya
UT		     Universal time
UWM	     University of Warmia-Mazury
VTEC	     Vertical Total Electron Content
WHU	     Wuhan University

# Appendix A - TEC maps sources

## A.1.1. IGS (International GNSS Service)

The International GNSS Service (IGS) was established in 1998 with the objective of generating reliable global vertical Total Electron Content (VTEC) maps [40]. It supplies combined Global Ionospheric Maps (GIM), which are obtained as a simple weighted mean of the VTEC maps produced by the available Ionospheric Associate Analysis Centers (IAAC), also called IGS Working Groups. There are currently eight IAACs: Center for Orbit Determination in Europe (CODE), European Space Agency (ESA), National Aeronautics Space Administration (NASA) Jet Propulsion Laboratory (JPL), Universitat Politècnica de Catalunya (UPC), Natural Resources Canada (NRCan), Chinese Academy of Sciences (CAS), Wuhan University (WHU) and Operational Tool for Ionospheric Mapping And Prediction/Deutsches Geodätisches Forschungsinstitut-Technische Universität München (OPTIMAP/DGFI-TUM). The NRCan, CAS and WHU IAACs started contributions in 2016, the OPTIMAP/DGFI-TUM started contributions in 2018, and the remaining, since 1998. These GIMs have been combined since 2007 by the University of Warmia-Mazury (UWM) to generate the IGS combined GIM [6,41].

The IGS VTEC combined maps in the IONosphere map Exchange (IONEX) format include a “final solution” product with ~11 days latency and weekly updates, and a “rapid solution” product with less than 24-hour latency and daily updates. The reliability and accuracy of the combined IGS GIM primarily depend on the fair evaluation of the consistency and accuracy of individual GIMs, provided by the different centers [4,5].

Further information about these maps is detailed below:

- Product: IGS combined GIM (VTEC maps);

- Temporal coverage: since 2/6/1998 (according to data repository);

- Latency: “final solution” with ~11 days; “rapid solution” with less than 24 hours [4];

- Single layer shell height: 450 km (according to IONEX data information);

- GNSS constellations: GPS and GLONASS [4];

- GNSS stations: IGS (international) (https://igs.org/faq/#igs-network-stations);

- Data format: IONEX [42];

- Data repository: https://cddis.nasa.gov/archive/gnss/products/ionosphere/.

## A.1.2. INPE/EMBRACE

The program EMBRACE, that stands for Studying and Monitoring of the Brazilian Space Weather (in Portuguese), is run by the Brazilian National Institute for Space Research (INPE). Ionospheric TEC maps over South America are generated for space weather applications. The spatial resolution of these maps ranges from 50 to 100 km in southeastern Brazil, 200 to 300 km in the northeastern region, and exceeds 500 km in the Amazon. In regions with high receiver density the spatial resolution can be improved to 0.5° x 0.5°, but out of the standard dissemination of TEC maps. EMBRACE TEC maps are updated every 10 minutes, with a processing delay of 12 hours. The satellite data is gathered by more than 140 monitoring stations, via GPS satellites constellation [1,2].

Further information about these maps is detailed below:

- Product: Regional Ionospheric Maps (RIM) (VTEC maps);

- Temporal coverage: since 11/24/2012 (according to data repository);

- Latency: 12 hours [2];

- Single layer shell height: 400 km (according to IONEX data information);

- GNSS constellations: GPS [2];

- GNSS stations: IGS (international), Brazilian Institute of Geography and Statistics (IBGE)/Brazilian Network for Continuous Monitoring (RBMC) (Brazil), Geophysical Institute of Peru (IGP)/Low latitude Ionospheric Sensor Network (LISN) (Peru) and National Geographical Institute (IGN)/Argentine Continuous Satellite Monitoring Network (RAMSAC) (Argentina) [2];

- Data format: IONEX (according to data repository) [2,42];

- Data repository: https://www2.inpe.br/climaespacial/SWMonitorUser/ (currently, some of the data is being migrated to https://embracedata.inpe.br/ionex/).

## A.1.3. UNLP-FCAGLP/MAGGIA

MAGGIA TEC maps are provided by the Meteorología espacial, Atmósfera terrestre, Geodesia, Geodinámica, diseño de Instrumental y Astrometría (MAGGIA) Laboratory of the La Plata National University (UNLP) - Faculty of Astronomical and Geophysical Sciences of the La Plata National University (FCAGLP), Argentina, being generated by a continuous atmospheric monitoring system that integrates real-time data from over 90 GNSS satellites - including the GPS, GLONASS, Galileo, and BDS. The related data is gathered from more than 200 monitoring stations spread across Central and South America, the Caribbean, Africa, Europe, and Antarctica. These stations are managed by various public institutions [8]. Sampling rate was increased from 15 to 10 minutes from 09/05/2024. Mapping was then changed from continuous-curvature spherical splines to weighted 2D binning using square root of distance [8-10].

Further information about these maps is detailed below:

- Product: Regional Ionospheric Maps (RIM) (VTEC maps);

- Temporal coverage: since 10/25/2018 (according to data repository);

- Latency: ~ 10 minutes [8];

- Single layer shell height: 450 km [8];

- GNSS constellations: GPS, GLONASS, Galileo and BDS [8];

- GNSS stations: Federal Agency for Cartography and Geodesy (BKG) (Germany), NASA (USA) (both in support to the IGS - international), IGS (international), National Institute of Geographic and Forest Information (IGN) (France), IBGE/RBMC (Brazil), IGN/RAMSAC (Argentina), Military Geographical Institute (IGM)/Active National Geodesic Network-Eastern Republic of Uruguay (REGNA-ROU) (Uruguay) and EarthScope (USA) [8]; also see info in this note;

- Data format: IONEX and Network Common Data Form (netCDF) [8,42,43];

- Data repository: http://wilkilen.fcaglp.unlp.edu.ar/ion/magn.

## A.1.4. University of Nagoya

The global TEC data were derived from GNSS observation data in Receiver INdependent EXchange (RINEX) format obtained from a lot of regional GNSS monitoring networks all over the world. These GNSS observation data were provided by many data providers.

The University of Nagoya/Institute for Space-Earth Environment Research (ISEE) (referred to as Nagoya for simplicity) generates four types of two-dimensional maps: absolute TEC, TEC difference ratio (rTEC), detrended TEC (dTEC), and rate of TEC index (ROTI) in geographic coordinates with a 5-minute time interval using RINEX files collected from over 9,300 GNSS receivers worldwide (as for January 2020). The grid size for the absolute TEC and rTEC maps is 0.50° x 0.50°, and for dTEC and ROTI maps is 0.25° x 0.25°. Additionally, grid data is smoothed using a 5 x 5 boxcar average for the TEC data in geographic latitude and longitude. Absolute TEC values are obtained using the bias estimation method proposed by [3]. The rTEC value is defined as the difference from the average TEC value of 10 geomagnetically quiet days each month, normalized by the absolute value of the average TEC.

Further information about these maps is detailed below:

- Product: Global Ionospheric Maps (GIM) (VTEC maps);

- Temporal coverage: since 10/10/1993 (according to data repository);

- Latency: information not found;

- Single layer shell height: 300 km (according to NetCDF data);

- GNSS constellations: GPS, GLONASS, Galileo, BDS and QZSS. Satellite-Based Augmentation System (SBAS) is also used to improve the GNSS signals [44];

- GNSS stations: data provider list of the RINEX files in Nov. 19, 2020: https://stdb2.isee.nagoya-u.ac.jp/GPS/GPS-TEC/gnss_provider_list.html;

- Data format: netCDF [43] (according to data repository);

- Data repository: https://stdb2.isee.nagoya-u.ac.jp/GPS/shinbori/AGRID2/nc/index.html.

Appendix B - Results

Table A1. Temporal coverage of TEC maps (DOY denotes day-of-the-year).

| Period | IGS | INPE | MAGGIA | Nagoya |
| --- | --- | --- | --- | --- |
| March/ 2022 | Complete | Complete | DOYs 68, 72 and 87 to 89 incomplete | Complete |
| June/ 2022 | Complete | Complete | DOYs 155, 175, 177, 179 and 180 incomplete | Complete |
| September/ 2022 | Complete | Complete | DOYs 244, 257, 259 and 263 incomplete | Complete |
| March/ 2024 | Complete | Complete | DOYs 82 to 85 incomplete | DOY 90 missing |
| June/ 2024 | Complete | Complete | DOYs 172, 179 and 182 incomplete | DOYs 178 and 179 incomplete |
| September/ 2024 | Complete | Complete | DOYs 259, 260 and 273 incomplete | DOYs 245 and 246 incomplete |
| December/ 2024 | Complete | DOY 348 incomplete and 366 missing | Complete | Complete |

Table A2. Joint correlations of TEC map sources using TF-120 (best value is marked in blue, and the worst, in red; all values are repeated in function of the source, for clarity).

| Source * | All months | 03/ 2022 | 06/ 2022 | 09/ 2022 | 03/ 2024 | 06/ 2024 | 09/ 2024 | 12/ 2024 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EMBRACE x Nagoya | 0.67 | 0.75 | 0.80 | 0.75 | 0.57 | 0.70 | 0.49 | 0.51 |
| EMBRACE x MAGGIA | 0.64 | 0.70 | 0.75 | 0.71 | 0.62 | 0.70 | 0.55 | 0.40 |
| EMBRACE x IGS | 0.62 | 0.67 | 0.71 | 0.69 | 0.58 | 0.69 | 0.54 | 0.42 |
| IGS x Nagoya | 0.61 | 0.66 | 0.72 | 0.62 | 0.57 | 0.67 | 0.50 | 0.53 |
| IGS x EMBRACE | 0.62 | 0.67 | 0.71 | 0.69 | 0.58 | 0.69 | 0.54 | 0.42 |
| IGS x MAGGIA | 0.95 | 0.93 | 0.97 | 0.96 | 0.96 | 0.98 | 0.92 | 0.80 |
| MAGGIA x Nagoya | 0.62 | 0.68 | 0.77 | 0.66 | 0.59 | 0.68 | 0.50 | 0.43 |
| MAGGIA x EMBRACE | 0.64 | 0.70 | 0.75 | 0.71 | 0.62 | 0.70 | 0.55 | 0.40 |
| MAGGIA x IGS | 0.95 | 0.93 | 0.97 | 0.96 | 0.96 | 0.98 | 0.92 | 0.80 |
| Nagoya x MAGGIA | 0.62 | 0.68 | 0.77 | 0.66 | 0.59 | 0.68 | 0.50 | 0.43 |
| Nagoya x IGS | 0.61 | 0.66 | 0.72 | 0.62 | 0.57 | 0.67 | 0.50 | 0.53 |
| Nagoya x EMBRACE | 0.67 | 0.75 | 0.80 | 0.75 | 0.57 | 0.70 | 0.49 | 0.51 |

* Reciprocal correlation (x and y, y and x) are equal, such correlations were repeated to enhance the clarity.

Table A3. Joint SSIM values of raster images generated from TF-120 maps (best value is marked in blue, and the worst, in red; all values are repeated in function of the source, for clarity).

| Source * | All months | 03/ 2022 | 06/ 2022 | 09/ 2022 | 03/ 2024 | 06/ 2024 | 09/ 2024 | 12/ 2024 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EMBRACE x Nagoya | 0.87 | 0.83 | 0.95 | 0.84 | 0.72 | 0.87 | 0.68 | 0.69 |
| EMBRACE x MAGGIA | 0.84 | 0.83 | 0.92 | 0.81 | 0.78 | 0.84 | 0.73 | 0.64 |
| EMBRACE x IGS | 0.82 | 0.81 | 0.91 | 0.79 | 0.77 | 0.84 | 0.73 | 0.67 |
| IGS x Nagoya | 0.77 | 0.76 | 0.90 | 0.76 | 0.66 | 0.81 | 0.64 | 0.59 |
| IGS x EMBRACE | 0.82 | 0.81 | 0.91 | 0.79 | 0.77 | 0.84 | 0.73 | 0.67 |
| IGS x MAGGIA | 0.96 | 0.92 | 0.98 | 0.94 | 0.94 | 0.97 | 0.88 | 0.85 |
| MAGGIA x Nagoya | 0.80 | 0.79 | 0.92 | 0.78 | 0.68 | 0.81 | 0.64 | 0.57 |
| MAGGIA x EMBRACE | 0.84 | 0.83 | 0.92 | 0.81 | 0.78 | 0.84 | 0.73 | 0.64 |
| MAGGIA x IGS | 0.96 | 0.92 | 0.98 | 0.94 | 0.94 | 0.97 | 0.88 | 0.85 |
| Nagoya x MAGGIA | 0.80 | 0.79 | 0.92 | 0.78 | 0.68 | 0.81 | 0.64 | 0.57 |
| Nagoya x IGS | 0.77 | 0.76 | 0.90 | 0.76 | 0.66 | 0.81 | 0.64 | 0.59 |
| Nagoya x EMBRACE | 0.87 | 0.83 | 0.95 | 0.84 | 0.72 | 0.87 | 0.68 | 0.69 |

* Reciprocal correlation (x and y, y and x) are equal, such correlations were repeated to enhance the clarity.

Table A4. Joint correlations of TEC map sources using TF-30, only for TEC values equal or above Q3 (best value is marked in blue, and the worst, in red).

| Source | All months | 03/2024 | 06/2024 | 09/2024 | 12/2024 |
| --- | --- | --- | --- | --- | --- |
| EMBRACE x Nagoya | 0.23 | 0.23 | 0.26 | 0.17 | 0.26 |
| Nagoya x EMBRACE | 0.12 | 0.13 | 0.76 | 0.11 | 0.15 |
| EMBRACE x MAGGIA | 0.36 | 0.36 | 0.46 | 0.33 | 0.30 |
| MAGGIA x EMBRACE | 0.12 | 0.18 | 0.05 | 0.10 | 0.16 |
| Nagoya x MAGGIA | 0.31 | 0.32 | 0.51 | 0.22 | 0.19 |
| MAGGIA x Nagoya | 0.22 | 0.19 | 0.40 | 0.13 | 0.13 |

Table A5. Joint SSIM values of raster images generated from TF-30 maps (best value is marked in blue, and the worst, in red; all values are repeated in function of the source, for clarity).

| Source * | All months | 03/2024 | 06/2024 | 09/2024 | 12/2024 |
| --- | --- | --- | --- | --- | --- |
| EMBRACE x Nagoya | 0.75 | 0.67 | 0.87 | 0.63 | 0.64 |
| EMBRACE x MAGGIA | 0.74 | 0.74 | 0.82 | 0.69 | 0.61 |
| MAGGIA x Nagoya | 0.67 | 0.64 | 0.80 | 0.59 | 0.53 |
| MAGGIA x EMBRACE | 0.74 | 0.74 | 0.82 | 0.69 | 0.61 |
| Nagoya x MAGGIA | 0.67 | 0.64 | 0.80 | 0.59 | 0.53 |
| Nagoya x EMBRACE | 0.75 | 0.67 | 0.87 | 0.63 | 0.64 |

* Reciprocal correlation (x and y, y and x) are equal, such correlations were repeated to enhance the clarity.

Table A6. Correlations for the unified/interpolated AE24, WS24, SE24 and SS24 TEC map sources  (best value is marked in blue, and the worst, in red; all values are repeated in function of the source, for clarity).

| Source * | All dates | AE24 | WS24 | SE24 | SS24 |
| --- | --- | --- | --- | --- | --- |
| EMBRACE x Nagoya | 0.58 | 0.59 | 0.52 | 0.44 | 0.73 |
| EMBRACE x MAGGIA | 0.64 | 0.84 | 0.61 | 0.41 | 0.60 |
| EMBRACE x IGS | 0.65 | 0.84 | 0.62 | 0.43 | 0.57 |
| MAGGIA x Nagoya | 0.49 | 0.67 | 0.56 | 0.10 | 0.53 |
| MAGGIA x EMBRACE | 0.64 | 0.84 | 0.61 | 0.41 | 0.60 |
| MAGGIA x IGS | 0.96 | 0.96 | 0.98 | 0.96 | 0.90 |
| IGS x Nagoya | 0.48 | 0.66 | 0.58 | 0.13 | 0.48 |
| IGS x EMBRACE | 0.65 | 0.84 | 0.62 | 0.43 | 0.57 |
| IGS x MAGGIA | 0.96 | 0.96 | 0.98 | 0.96 | 0.90 |
| Nagoya x MAGGIA | 0.49 | 0.67 | 0.56 | 0.10 | 0.53 |
| Nagoya x IGS | 0.48 | 0.66 | 0.58 | 0.13 | 0.48 |
| Nagoya x EMBRACE | 0.58 | 0.59 | 0.52 | 0.44 | 0.73 |

* Reciprocal correlation (x and y, y and x) are equal, such correlations were repeated to enhance the clarity.

Table A7. SSIM values for the raster images generated from the unified/interpolated AE24, WS24, SE24 and SS24 TEC maps (best value is marked in blue, and the worst, in red; all values are repeated in function of the source, for clarity).

| Source * | All dates | AE24 | WS24 | SE24 | SS24 |
| --- | --- | --- | --- | --- | --- |
| EMBRACE x Nagoya | 0.83 | 0.76 | 0.91 | 0.70 | 0.88 |
| EMBRACE x MAGGIA | 0.73 | 0.67 | 0.76 | 0.80 | 0.67 |
| EMBRACE x IGS | 0.73 | 0.66 | 0.75 | 0.82 | 0.65 |
| IGS x Nagoya | 0.67 | 0.60 | 0.76 | 0.70 | 0.60 |
| IGS x EMBRACE | 0.73 | 0.66 | 0.75 | 0.82 | 0.65 |
| IGS x MAGGIA | 0.94 | 0.90 | 0.95 | 0.97 | 0.92 |
| MAGGIA x Nagoya | 0.68 | 0.61 | 0.76 | 0.69 | 0.62 |
| MAGGIA x EMBRACE | 0.73 | 0.67 | 0.76 | 0.80 | 0.67 |
| MAGGIA x IGS | 0.94 | 0.90 | 0.95 | 0.97 | 0.92 |
| Nagoya x MAGGIA | 0.68 | 0.61 | 0.76 | 0.69 | 0.62 |
| Nagoya x IGS | 0.67 | 0.60 | 0.76 | 0.70 | 0.60 |
| Nagoya x EMBRACE | 0.83 | 0.76 | 0.91 | 0.70 | 0.88 |

* Reciprocal correlation (x and y, y and x) are equal, such correlations were repeated to enhance the clarity.

Figure A1. Original TEC maps AE24, WS24, SE24 and SS24, TEC maps (shown in each row from left to right), taken as Equinoxes and Solstices samples for 2024, generated by (from top to bottom) EMBRACE (1st row), IGS (2nd row), MAGGIA (3rd row) and Nagoya (4th row). In all maps, the continuous black line denotes the magnetic equator and the dashed lines, the dip latitudes spaced by 10°. The bottom-right corner of each map denotes the [minimum,maximum] range of its TEC values  and the name of the source.

Table A8. Correlations for the unified/interpolated AE24, WS24, SE24 and SS24 TEC map sources considering TEC values equal or above Q3 (best value is marked in blue, and the worst, in red).

| Source | All dates | AE24 | WS24 | SE24 | SS24 |
| --- | --- | --- | --- | --- | --- |
| EMBRACE x Nagoya | 0.09 | -0.14 | 0.20 | 0.07 | -0.05 |
| Nagoya x EMBRACE | 0.32 | 0.29 | -0.01 | 0.15 | 0.71 |
| EMBRACE x MAGGIA | 0.41 | 0.54 | 0.30 | 0.04 | 0.65 |
| MAGGIA x EMBRACE | -0.09 | 0.03 | -0.27 | -0.09 | -0.01 |
| IGS x EMBRACE | -0.18 | 0.18 | -0.32 | -0.27 | -0.29 |
| EMBRACE x IGS | 0.43 | 0.56 | 0.31 | 0.09 | 0.67 |
| IGS x MAGGIA | 0.72 | 0.43 | 0.97 | 0.74 | 0.15 |
| MAGGIA x IGS | 0.76 | 0.40 | 0.90 | 0.72 | 0.84 |
| Nagoya x IGS | 0.45 | 0.53 | 0.64 | 0.04 | 0.50 |
| IGS x Nagoya | -0.10 | 0.29 | 0.29 | -0.29 | -0.59 |
| Nagoya x MAGGIA | 0.51 | 0.62 | 0.62 | 0.01 | 0.65 |
| MAGGIA x Nagoya | 0.04 | 0.12 | 0.34 | -0.01 | -0.29 |

Table A9. TEC values for EMBRACE, MAGGIA and Gopi Seemala (± standard deviation) corresponding to the seven chosen GNSS stations, for September 27th, 2024 at 00:50, 01:00 and 01:10 UT (values in TECU).

| Source | 00:50 UT | 01:00 UT | 01:10 UT |
| --- | --- | --- | --- |
| Gopi (BRAZ) | 91.93 ± 8.91 | 92.28 ± 7.67 | 93.38 ± 9.55 |
| EMBRACE (BRAZ) | 72.52 | 71.86 | 67.75 |
| MAGGIA (BRAZ) | 138.74 | 133.04 | 129.11 |
| Gopi (CUIB) | 73.90 ± 11.18 | 73.87 ± 10.38 | 74.22 ± 10.51 |
| EMBRACE (CUIB) | 36.89 | 43.34 | 42.93 |
| MAGGIA (CUIB) | 75.94 | 111.90 | 85.23 |
| Gopi (NAUS) | 19.83 ± 2.12 | 19.68 ± 1.55 | 19.47 ± 2.55 |
| EMBRACE (NAUS) | 28.84 | 32.11 | 34.07 |
| MAGGIA (NAUS) | 40.93 | 39.25 | 44.92 |
| Gopi (POAL) | 14.77 ± 4.29 | 15.80 ± 3.82 | 16.92 ± 3.35 |
| EMBRACE (POAL) | 33.90 | 33.01 | 34.13 |
| MAGGIA (POAL) | 29.80 | 27.86 | 26.85 |
| Gopi (SALU) | 21.98 ± 6.73 | 20.75 ± 6.62 | 19.45 ± 7.05 |
| EMBRACE (SALU) | 39.31 | 41.23 | 34.59 |
| MAGGIA (SALU) | 38.69 | 42.25 | 43.50 |
| Gopi (SAVO) | 101.46 ± 7.79 | 98.65 ± 8.53 | 94.63 ± 5.26 |
| EMBRACE (SAVO) | 69.32 | 73.81 | 73.59 |
| MAGGIA (SAVO) | 121.90 | 133.17 | 127.19 |
| Gopi (SJSP) | 30.89 ± 19.69 | 30.06 ± 11.62 | 29.80 ± 7.92 |
| EMBRACE (SJSP) | 31.64 | 30.80 | 28.02 |
| MAGGIA (SJSP) | 51.12 | 49.78 | 47.96 |

Table A10.  Overall values for the set of metrics calculated using TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps for the 27 selected GNSS stations, using Gopi Seemala values as a reference, for all times of day and all days of December, 2024, discriminated by the two clusters of stations and by quartiles (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE | Cluster |
| --- | --- | --- | --- | --- | --- | --- |
| EMBRACE (Q-lower) | 4.48 | 5.63 | 0.33 | 0.36 | -0.44 | Amazonian |
| EMBRACE (Q-upper) | 6.06 | 11.58 | 0.41 | 0.34 | -0.58 | Amazonian |
| EMBRACE (Q-inter) | 7.37 | 11.43 | 0.81 | 0.86 | 0.67 | Amazonian |
| EMBRACE (Q-lower) | 9.83 | 11.79 | 0.58 | 0.45 | -0.59 | Southeastern |
| EMBRACE (Q-upper) | 14.59 | 16.26 | 0.42 | 0.57 | 0.12 | Southeastern |
| EMBRACE (Q-inter) | 16.01 | 18.88 | 0.72 | 0.69 | 0.19 | Southeastern |
| IGS (Q-lower) | 13.58 | 14.40 | 0.36 | 0.37 | -1.10 | Amazonian |
| IGS (Q-upper) | 12.65 | 13.57 | 0.67 | 0.78 | 0.52 | Amazonian |
| IGS (Q-inter) | 13.30 | 14.41 | 0.94 | 0.96 | 0.62 | Amazonian |
| IGS (Q-lower) | 21.18 | 22.09 | 0.68 | 0.57 | -1.32 | Southeastern |
| IGS (Q-upper) | 19.76 | 20.61 | 0.48 | 0.73 | 0.37 | Southeastern |
| IGS (Q-inter) | 22.25 | 23.43 | 0.80 | 0.84 | 0.26 | Southeastern |
| MAGGIA (Q-lower) | 14.22 | 15.39 | 0.30 | 0.25 | -1.59 | Amazonian |
| MAGGIA (Q-upper) | 23.96 | 26.68 | 0.16 | 0.13 | -2.08 | Amazonian |
| MAGGIA (Q-inter) | 17.91 | 19.94 | 0.80 | 0.78 | 0.36 | Amazonian |
| MAGGIA (Q-lower) | 20.05 | 21.19 | 0.48 | 0.37 | -1.41 | Southeastern |
| MAGGIA (Q-upper) | 25.06 | 27.15 | 0.18 | 0.20 | -1.32 | Southeastern |
| MAGGIA (Q-inter) | 25.03 | 26.69 | 0.61 | 0.55 | -0.17 | Southeastern |
| Nagoya (Q-lower) | 6.86 | 8.13 | 0.23 | 0.25 | -1.03 | Amazonian |
| Nagoya (Q-upper) | 35.06 | 47.23 | 0.04 | 0.04 | -4.84 | Amazonian |
| Nagoya (Q-inter) | 18.73 | 26.11 | 0.44 | 0.60 | 0.13 | Amazonian |
| Nagoya (Q-lower) | 10.60 | 12.86 | 0.45 | 0.27 | -1.16 | Southeastern |
| Nagoya (Q-upper) | 18.12 | 23.55 | 0.20 | 0.12 | -2.32 | Southeastern |
| Nagoya (Q-inter) | 17.79 | 21.50 | 0.43 | 0.41 | -0.34 | Southeastern |

Table A11. Overall values for the set of metrics calculated using TEC values extracted from the  EMBRACE, IGS, MAGGIA and Nagoya maps for the 27 selected GNSS stations, using Gopi Seemala values as a reference, for 18 UT of all days of December, 2024 discriminated by quartiles (MAE and RMSE values in TECU).

| Source | MAE | RMSE | ρ | TSS | KGE |
| --- | --- | --- | --- | --- | --- |
| EMBRACE (Q-lower) | 16.05 | 18.39 | 0.05 | 0.28 | -0.63 |
| EMBRACE (Q-upper) | 5.10 | 6.50 | 0.37 | 0.53 | 0.09 |
| EMBRACE (Q-inter) | 9.20 | 14.41 | 0.06 | 0.10 | -2.58 |
| IGS (Q-lower) | 25.10 | 25.97 | 0.22 | 0.60 | 0.07 |
| IGS (Q-upper) | 10.66 | 11.83 | 0.46 | 0.65 | 0.30 |
| IGS (Q-inter) | 16.07 | 17.04 | 0.25 | 0.45 | -0.12 |
| MAGGIA (Q-lower) | 30.09 | 31.41 | -0.03 | 0.10 | -2.35 |
| MAGGIA (Q-upper) | 22.12 | 25.18 | 0.17 | 0.08 | -3.20 |
| MAGGIA (Q-inter) | 23.83 | 24.99 | 0.14 | 0.13 | -2.08 |
| Nagoya (Q-lower) | 24.25 | 29.84 | -0.03 | 0.05 | -4.03 |
| Nagoya (Q-upper) | 31.60 | 46.00 | 0.03 | 0.02 | -7.06 |
| Nagoya (Q-inter) | 23.09 | 34.32 | -0.09 | 0.02 | -8.21 |

# References

1. Takahashi, H.; Wrasse, C.M.; Otsuka, Y.; Ivo, A.; Gomes, V.; Paulino, I.; Medeiros, A.F.; Denardini, C.M.; Sant’Anna, N.; Shiokawa, K. Plasma bubble monitoring by TEC map and 630 nm airglow image. J. Atmos. Sol.-Terr. Phys 2015, 130-131, 151-158. https://doi.org/10.1016/j.jastp.2015.06.003

2. Takahashi, H.; Wrasse, C.M.; Denardini, C.M.; Pádua, M.B.; de Paula, E.R.; Costa, S.M.A.; Otsuka, Y.; Shiokawa, K.; Monico, J.F.G.; Ivo, A. Ionospheric TEC Weather Map Over South America. Space Weather 2016, 14, 937-949. https://doi.org/10.1002/2016SW001474

3. Otsuka, Y.; Ogawa, T.; Saito, A.; Tsugawa, T.; Fukao, S.; Miyazaki, S. A new technique for mapping of total electron content using GPS network in Japan. Earth Planets Space 2002, 54, 63-70. https://doi.org/10.1186/BF03352422

4. Hernández-Pajares, M.; Juan, J.M.; Sanz, J.; Orus, R.; Garcia-Rigo, A.; Feltens, J.; Komjathy, A.; Schaer, S.C.; Krankowski, A. The IGS VTEC maps: A reliable source of ionospheric information since 1998. J. Geod 2009, 83, 263-275. https://doi.org/10.1007/s00190-008-0266-1

5. Hernández-Pajares, M.; Roma-Dollase, D.; Krankowski, A.; Garcia-Rigo, A.; Orús-Pérez, R. Methodology and consistency of slant and vertical assessments for ionospheric electron content models. J. Geod 2017, 91, 1405-1414. https://doi.org/10.1007/s00190-017-1032-z

6. Roma-Dollase, D.; Hernández-Pajares, M.; Krankowski, A.; Kotulak, K.; Ghoddousi-Fard, R.; Yuan, Y.; Li, Z.; Zhang, H.; Shi, C.; Wang, C. Consistency of seven different GNSS global ionospheric mapping techniques during one solar cycle. J. Geod 2018, 92, 691-706. https://doi.org/10.1007/s00190-017-1088-9

7. Hernández-Pajares, M.; Roma-Dollase, D. Correction to: “Methodology and consistency of slant and vertical assessments for ionospheric electron content models” and to “Consistency of seven different GNSS global ionospheric mapping techniques during one solar cycle”. J. Geod 2020, 94, 38-39. https://doi.org/10.1007/s00190-020-01346-y

8. Mendoza, L.P.O.; Meza, A.M.; Paz, J.M.A. A Multi-GNSS, Multifrequency and Near-Real-Time Ionospheric TEC Monitoring System for South America. Space Weather 2019, 17, 654-661. https://doi.org/10.1029/2019SW002187

9. Mendoza, L.P.O.; Meza, A.M.; Paz, J.M.A. Technical note on the multi-GNSS, multi-frequency and near real-time ionospheric TEC monitoring system for South America. EarthArXiv 2019, 1-11. https://doi.org/10.31223/osf.io/3vts6

10. Mendoza, L.P.O.; Meza, A.M.; Paz, J.M.A. Near-real-time VTEC maps: New contribution for Latin America Space Weather. Adv. Space Res 2020, 65, 2235-2246. https://doi.org/10.1016/j.asr.2019.08.045

11. Sreeja, V.; Aquino, M.; Elmas, Z. G. Impact of ionospheric scintillation on GNSS receiver tracking performance over Latin America: Introducing the concept of tracking jitter variance maps. Space Weather 2011, 9, 1-6. https://doi.org/10.1029/2011SW000707

12. Seemala, G. K. Estimation of ionospheric total electron content (TEC) from GNSS observations. In Atmospheric Remote Sensing: Principles and Applications, 1st ed.; Singh, A.K., Tiwari, S., Eds.; Elsevier: [s.l.], 2023; pp. 154-196. https://doi.org/10.1016/B978-0-323-99262-6.00022-5

13. Hapgood, M. Technological Impacts of Space Weather. In Geomagnetism, Aeronomy and Space Weather: A Journey from the Earth’s Core to the Sun, 1st ed.; Mandea, M., Korte, M., Yau, A., Petrovsky, E., Eds.; Cambridge University Press: Cambridge, UK, 2019; pp. 203-218.

14. Muella, M.T.A.H.; de Paula, E.R.; Mitchell, C.N.; Kintner, P.M.; Paes, R.R.; Batista, I.S. Tomographic imaging of the equatorial and low-latitude ionosphere over central-eastern Brazil. Earth Planet Sp 2011, 63, 129-138. https://doi.org/10.5047/eps.2010.12.011

15. Grewal, M.S.; Andrews, A.P.; Bartone, C.G. Global Navigation Satellite Systems, Inertial Navigation, and Integration, 4th ed.; Wiley: Hoboken, USA, 2020.

16 Fejer, B.G.; Kelley, M. C. Ionospheric irregularities. Rev Geophys 1980, 18, 401-454. https://doi.org/10.1029/RG018i002p00401

17. Makela, J. J. A review of imaging low-latitude ionospheric irregularity processes. J. Atmos. Sol.-Terr. Phys. 2006, 68, 1441-1458. https://doi.org/10.1016/j.jastp.2005.04.014

18. Peng, Y.; Scales, W.A.; Hartinger, M.D.; Xu, Z.; Coyle, S. Characterization of multi-scale ionospheric irregularities using ground-based and space-based GNSS observations. Satell Navig 2021, 2, 1-21. https://doi.org/10.1186/s43020-021-00047-x

19. Picanço, G.A.S.; Denardini, C.M.; Nogueira, P.A.B.; Resende, L.C.A.; Carmo, C. S.; Chen, S. S.; Barbosa-Neto, P. F.; Romero-Hernandez, E. Study of the equatorial and low-latitude total electron content response to plasma bubbles during solar cycle 24–25 over the Brazilian region using a Disturbance Ionosphere indeX. Ann. Geophys 2022, 40, 503-517. https://doi.org/10.5194/angeo-40-503-2022

20. Oliveira, C.B.A.D.; Espejo, T.M.S.; Moraes, A.; Costa, E.; Sousasantos, J.; Lourenco, L.F.D.; Abdu, M.A. Analysis of Plasma Bubble Signatures in Total Electron Content Maps of the Low-Latitude Ionosphere: A Simplified Methodology. Surv Geophys 2020, 41, 897–931. https://doi.org/10.1007/s10712-020-09584-7

21. Abdu, M.A. Day-to-day and short-term variabilities in the equatorial plasma bubble/spread F irregularity seeding and development. Prog. Earth Planet. Sci 2019, 6, 1-22. https://doi.org/10.1186/s40645-019-0258-1

22. Balan, N.; Liu, L.; Le, H. A brief review of equatorial ionization anomaly and ionospheric irregularities. Earth Planet. Phys. 2018, 2, 257-275. https://doi.org/10.26464/epp2018025

23. Dierendonck, A.V.; Klobuchar, J.; Hua, Q. Ionospheric Scintillation Monitoring Using Commercial Single Frequency C/A Code Receivers. In Proceedings of the 6th International Technical Meeting of the Satellite Division of The Institute of Navigation (ION GPS 1993), USA, 22-24 September 1993.

24. de Paula, E.R.; Martinon, A.R.F.; Moraes, A.O.; Carrano, C.; Neto, A.C.; Doherty, P.; Groves, K.; Valladares, C.E.; Crowley, G.; Azeem, I. Performance of 6 Different Global Navigation Satellite System Receivers at Low Latitude Under Moderate and Strong Scintillation. Earth Space Sci. 2021, 8, 1-22. https://doi.org/10.1029/2020EA001314

25. Hofmann-Wellenhof, B.; Lichtenegger, H.; Wasle, E. GNSS-Global Navigation Satellite Systems: GPS, GLONASS, Galileo, and More, 1st ed.; Springer: Vienna, Austria, 2008.

26. Jonah, O.F. Analysis of Total Electron Content (TEC) variations obtained from GPS data over South America. Master’s Thesis, Instituto Nacional de Pesquisas Espaciais, São José dos Campos, Brazil, 2013.

27. Kenney, J. F.; Keeping, E. S. Linear Regression and Correlation. In Mathematics of Statistics, 3rd ed.; Van Nostrand: Princeton, USA, 1962; pp. 252-285.

28. Fisher, R. A. Frequency distribution of the values of the correlation coefficient in samples from an indefinitely large population. Biometrika 1915, 10, 507-521. https://doi.org/10.2307/2331838

29. Fisher, R. A. On the “probable error” of a coefficient of correlation deduced from a small sample. Metron 1921, 1, 3–32.

30. Wang, Z.; Bovik, A.C.; Sheikh, H.R.; Simoncelli, E.P. Image quality assessment: From error visibility to structural similarity. IEEE Trans. Image Process 2004, 13, 600–612. https://doi.org/10.1109/TIP.2003.819861

31. Taylor, K.E. Summarizing multiple aspects of model performance in a single diagram. J Geophys Res 2001, 106, 7183–7192. https://doi.org/10.1029/2000JD900719

32. Gupta, H.V.; Kling, H.; Yilmaz, K.K.; Martinez, G.F. Decomposition of the mean squared error and NSE performance criteria: Implications for improving hydrological modelling. J. Hydrol. 2009, 377, 80-91. https://doi.org/10.1016/j.jhydrol.2009.08.003

33. Rajana, S.S.K.; Panda, S.K.; Jade, S.; Vivek, C.G. Morphology of equatorial F-region irregularities over the Indian longitude sector using GPS-derived ROTI observations. Adv. Space Res 2024, 74, 3361–3377. https://doi.org/10.1016/j.asr.2024.06.039

34. Vankadara, R.K.; Panda, S.K.; Amory-Mazaudier, C.; Fleury, R.; Devanaboyina, V.R.; Pant, T.K.; Jamjareegulgarn, P.; Haq, M.A.; Okoh, D.; Seemala, G.K. Signatures of Equatorial Plasma Bubbles and Ionospheric Scintillations from Magnetometer and GNSS Observations in the Indian Longitudes during the Space Weather Events of Early September 2017. Remote Sens 2022, 14, 1-26. https://doi.org/10.3390/rs14030652

35. Martinon, A.R.F.; Stephany, S.; de Paula, E.R. A new approach for the generation of real-time GNSS low-latitude ionospheric scintillation maps. J. Space Weather Space Clim 2023, 13, 1-17. https://doi.org/10.1051/swsc/2023015

36. de Paula, E.R.; Monico, J.F.G.; Tsuchiya, I.H.; Valladares, C.E.; Costa, S.M.A.; Marini-Pereira, L.; Vani, B.C.; Moraes, A.O. A Retrospective of Global Navigation Satellite System Ionospheric Irregularities Monitoring Networks in Brazil. J. Aerosp. Technol. Manag 2023, 15, 1-17. https://doi.org/10.1590/jatm.v15.1288

37. Marini-Pereira, L.; Lourenço, L. F. D.; Sousasantos, J.; Moraes, A.O.; Pullen, S. Regional Ionospheric delay mapping for low-latitude environments. Radio Sci 2020, 55, 1-16. https://doi.org/10.1029/2020RS007158

38. Monico, J.F.G.; de Paula, E.R.; Moraes, A.O.; Costa, E.; Shimabukuro, M.H.; Alves, D.M.B.; Souza, J.R.; Camargo, P.O.; Prol, F.S.; Vani, B.C. The GNSS NavAer INCT Project Overview and Main Results. J. Aerosp. Technol. Manag 2022, 14, 1-23. https://doi.org/10.1590/jatm.v14.1249

39. Brazilian Institute of Geography and Statistics. Available online: https://geoftp.ibge.gov.br/informacoes_sobre_posicionamento_geodesico/rbmc/cartogramas/RBMC_2024.pdf (accessed on 27 September 2025).

40. Feltens, J.; Schaer, S. IGS Products for the Ionosphere. In Proceedings of the IGS Analysis Center Workshop, Darmstadt, Germany, 9-11 February 1998.

41. Panda a, S.K.; Harikaa, B.; Vineetha, P.; Kumar Dabbakutib, J.R.K.; Akhila, S.; Srujanaa, G. Validity of Different Global Ionospheric TEC Maps over Indian Region. In Proceedings of the 2021 3rd International Conference on Advances in Computing, Communication Control and Networking (ICAC3N), Greater Noida, India, 17–18 December 2021; pp. 1749–1755. https://doi.org/10.1109/ICAC3N53548.2021.9725568

42. Schaer, S.; Gurtner, W.; Feltens, J. IONEX: The IONosphere Map EXchange Format Version 1, February 25, 1998. In Proceedings of the IGS Analysis Center Workshop, Darmstadt, Germany, 9-11 February 1998.

43. Rew, R.; Davis, G. NetCDF: an interface for scientific data access. IEEE Comput. Graph. Appl 1990, 10, 76-82. https://doi.org/10.1109/38.56302

44. Otsuka, Y. (Nagoya University, Nagoya, Aichi, Japan). Personal communication, 2025.

Disclaimer/Publisher’s Note: The statements, opinions and data contained in all publications are solely those of the individual author(s) and contributor(s) and not of MDPI and/or the editor(s). MDPI and/or the editor(s) disclaim responsibility for any injury to people or property resulting from any ideas, methods, instructions or products referred to in the content.

https://igs.org/products/#ionosphere

https://www2.inpe.br/climaespacial/portal/tec-map-sobre/ / https://www2.inpe.br/climaespacial/portal/tec-map-inicio/

https://www.maggia.unlp.edu.ar/productos_maggia_alta

https://stdb2.isee.nagoya-u.ac.jp/GPS/GPS-TEC/index.html

https://www.timeanddate.com/calendar/seasons.html

https://seemala.blogspot.com/2024/04/gps-tec-analysis-program-version-35.html

http://stdb2.isee.nagoya-u.ac.jp/GPS/GPS-TEC/gnss_provider_list.html

https://igs.org/products/#ionosphere

https://www2.inpe.br/climaespacial/portal/en/

http://wilkilen.fcaglp.unlp.edu.ar/ion/magn/CHANGES.TXT

http://wilkilen.fcaglp.unlp.edu.ar/ion/latest.png

https://www.ibge.gov.br/en/geosciences/geodetic-positioning/geodetic-networks/20079-brazilian-network-for-continuous-monitoring-gnss-systems.html?lang=en-GB&t=sobre

https://www.ibge.gov.br/geociencias/informacoes-sobre-posicionamento-geodesico/rede-geodesica/16258-rede-brasileira-de-monitoramento-continuo-dos-sistemas-gnss-rbmc.html?=&t=dados-diarios-e-situacao-operacional

Geographical coordinates are available in document “Geographical_coordinates_of_the_27_selected_GNSS_stations.pdf” in Zenodo at https://doi.org/10.5281/zenodo.15453941

The complete set of metrics for each station and for each source is available in document “test_case_06-error_estimation_by_network_and_station.csv” in Zenodo at https://doi.org/10.5281/zenodo.15453941